HAMILTON HALLMARK

TECHNICAL SUPPORT CENTER

900 MHz Transceiver Application Note

Author: Sal Navidi

June 16, 1996

Disclaimer

Hamilton Hallmark makes no warranty for the use of this Reference Design and assumes no responsibility for any errors which may appear in any of the information contained in this Reference Design Kit, nor does it make a commitment to update the information contained herein.

Hamilton Hallmark retains the right to make changes at any time, without notice, to the information contained in this Reference Design Kit.

Introduction:

Wireless communication has become increasingly popular in the recent years.  The Industrial, Scientific and Medical (ISM) bands are allocated for non licensed applications and are used for majority of the wireless applications.  Presently, three bands are designated by FCC for ISM applications.  The first ISM band is at 902 to 928 MHz.  This band is heavily used for wireless applications.  The next band allocation is between 2400 to 2500 MHz.  This band is becoming increasingly popular and will be used for the wireless LAN 802.11 protocol.  The last ISM band spans over 5725 to 5850 MHz.  Presently there are very little activity on this band.  This may be attributed to lack of commercial building block chips(e.g. RFIC).

A low cost  half duplex FSK Transceiver operating at 906.5 MHz has been built and tested at Hamilton Hallmark Technical Support Center.   Reliable link performance has been observed at 9.6 Kbaud, for a line-of-sight distance of 50 ft.   The main criterion for the development of this transceiver was to design a wireless RS-232 link for PC to PC applications and data acquisition.

This wireless transceiver consists of a receiver , transmitter, receive/transmit switch and a pre-selector filter. The receiver consists of a low noise amplifier, 1st mixer, 1st local oscillator, 1st IF filter, second mixer, 2nd local oscillator, limiter, and a demodulator.  Dual conversion ( two stages of down conversion) is utilized for the main purpose of image frequency rejection.   The transmitter usually consists of a carrier and a modulator (it is also possible to modulate an IF signal and up-convert to the final carrier frequency). Various forms of modulation are possible.  Frequency Shift Keying (FSK) modulation was used in this transceiver.  This method is less susceptible to amplitude variations than the Pulse Amplitude Modulation (PAM).  The demodulation of  FSK is not as complicated as other types of digital modulation schemes such as Binary Phase Shift Keying (BPSK) or Quadrature Phase Shift Keying  (QPSK) and can be easily achieved with off-the-shelf FM receiver chips.  Signal to noise ratio improvement is also achieved in FM systems when the maximum deviation frequency is greater than the baud rate.

Three are three common methods of FSK modulation.   These are I-Q modulation, direct modulation of the a Voltage Controlled Oscillator (VCO) in a phase lock loop, and pulling (changing the load of ) the reference Crystal oscillator in the phase lock loop.  The I-Q modulation for FSK generation is some what complex and is usually used in cases where the transmit spectrum has to fit in a pre-defined (tight) bandwidth (e.g. multi channel systems).  External modulation of the VCO  is the most common method of FSK modulation.  The requirement for this method is that the data rate  has to be higher than the loop bandwidth of the phase lock loop so that the loop can not correct the frequency deviation induced by the data.   The draw back to this method is when a long string of 'one's are modulating the VCO, the phase lock loop has enough time to correct for the disturbance and adjust for the frequency deviation.   The practical fix for this situation is to scramble (randomize) the data so a bunch of 'ones' don't occur consecutively.  The third method of FSK modulation varies the reference crystal frequency therefore changing the locked VCO frequency.  The crystal frequency is altered by changing the shunt load capacitance to the crystal   This is achieved by placing a varactor in parallel with the crystal.  The varactor is modulated with the input data to change its capacitance.  The drawback to this method is that the crystal frequency usually can not be pulled more that 100 PPM.  This limits the maximum achievable frequency deviation and is only used in narrow band FM applications.   Another draw back to this method is that the loop band width should be several times faster than the data rate.  This is needed so that the VCO can lock to the reference frequency within a fraction of a bit rate.   Higher loop band width also causes more phase noise and less attenuation of the spurious side bands.  The above disadvantages limit the use of this method for high data rate modulation.   The  major advantage of this method is that the there is no low-baud rate limitation.  This makes this method viable for low baud rate (e.g. 9600 Kbaud) PC applications.

System Description:

This  906 MHz transceiver consist of a ceramic band pass  filter ( Motorola KFF6140A),  RF switch (Motorola MRFIC2003 ), and the receive and transmit sections (figure 1).

The dual conversion receiver utilizes the Motorola MRFIC2001(Low Noise Amplifier and the Mixer).  The first Local Oscillator (LO) operates at 794.5 MHz and is constructed using Motorola MC13176.   The first IF(112 MHz) is filtered and is fed to the  MC13156 which performs the second conversion (LO at 122.7 MHz), IF amplification and quadrature detection.  The maximum sensitivity was measured to be -80 dBm without any shielding of the receiver.

The transmitter consists of a single MC13176 chip utilizing a 28.328125 MHz Reference Crystal (a divide by 32 prescaler resides in the 13176).  The transmit power output was measured at -8 dBm (6 dB lower than the maximum allowed by FCC).  Two FSK modulation schemes were experimented   The direct modulation of the Current Controlled Oscillator inside the loop was found to be reliable for maximum baud rate of 38.4 Kbaud.  The maximum  frequency deviation was set was at 90 KHz.  The low-baud rate limit was determined to be 2400 baud.  The limit on  the number of consecutive 'one' s transmitted was measured to be at two bytes at 38.4 Kbaud without the uses of scrambling techniques.   Utilization of varactor modulation of the reference crystal resulted in reliable link performance at 9600 baud.   This was achieved with a deviation frequency of 70 KHz.  As mentioned earlier there is no low-baud rate limit on this technique.

Practical Considerations:

The following describes detailed implementation of the 906 MHz RDK.   Other information can also be found in reference 1 and 2.

Layout:

The main consideration for an RF layout is to have low impedance Ground and Vcc connections to all elements.  This is achieved by using Ground and Vcc planes rather than lines.  For example, the unused portion of the component side on a double sided board can be used as the Vcc plane and the bottom side as the ground plane.   Minimizing the inductance between the above planes is also very critical in RF design.  For this reason all Ground pins must be bypassed as close as possible to the actual IC pin. Since surface mount components are used the ground connection must be brought up to the top using Via holes.  The diameter of these Via's should be as large as practical to reduce the associated inductance.

Transmitter:

The transmitter is built around MC13176.   Internal to this chip are a Current Controlled Oscillator (CCO) and a Phase Lock Loop.  The most logical method for initial test of this chip is to disconnect the oscillator control input (pin 6).   This enables the debug of the oscillator and the tank circuitry.  The variable inductor (L6) can be adjusted to tune the oscillator close to 906.5 MHz.  The CCO transfer function (MHz / µAcan be  measured by injecting a small current (using a variable power supply with a series resistor) into pin 6.  This value can be used to calculate the loop filter components (please refer to the MC13156 data sheet for loop filter calculations).  Pin 6 is connected after the CCO has been tuned.  This will lock the CCO to the crystal frequency.   The initial frequency of the transmitter ( representing 'zeros'),  is set by adjustment of the load capacitance of the crystal.  Grounding the DATAIN line will result in a 1V reverse bias on the MMBV609 Varactor which yields approximately 44 pf of capacitance.   Capacitors C9 and C10 are modified to set the frequency within  30 KHz of 906.5 MHz.  The FM deviation of the transmitter (transmitting 'one's) is adjusted by connecting the DATAIN line to Vcc thus resulting in a varactors capacitance of approximately 20 pf.  Components C103 , R12 and R52 can be altered to fine tune the deviation frequency.  It should be noted that adjustment of the above components can also affect the initial frequency setting.

Preselector Filter:

The front-end Ceramic filter should be soldered in with low temp solder and or put in place with silver epoxy.  Excessive heat can damage this part.

Antenna Switch:

The antenna switch MRFIC2003 operated without any complication once C102 was added to block the DC.

LNA and Mixer:

The LNA and Mixer reside within MRFIC2001.   No complication was noticed with the initial testing of this part.

Local Oscillator:

The first Local Oscillator (LO) is made with MC13176.   It is basically the same circuit as the transmitter without the FM modulation circuit and a simpler loop filter.  A capacitor (C12) is added from pin 1 to ground to lower the inductance value needed for operation at 794.5 MHz.   The CCO should first be  tuned to the proper frequency with the control pin disconnected.  The loop filter is connected  and C19 modified to fine tuning the frequency.  The LO power level should be set in the range of  -7 to -10 dBm with the proper selection of R26.  This is done to minimize the LO radiation and coupling to other parts of the circuit.

FM Receiver:

The FM receiver chip used in this reference design is MC13156.  This chip contains a mixer, amplifier, limiter, quadrature detector and a compactor.  A fifth overtone crystal oscillator with a transistor internal to the chip make up the 122.7 MHz Local Oscillator.  Variable inductor L5 is used to tune this Oscillator.  The quadrature detector demodulates the FM signal.  A detailed discussion of FM demodulation and quadrature detection is covered in reference 2.  The raw output of the quadrature demodulator (Pin 14) can be used to adjust L3 for the largest peak to peak output.

References:

1-    H. Swanson, "An FSK Modulation Data Receiver for 900 MHz Using Low Cost Monolithic Chip Set", RF EXPO West , January 1994

2 -   S. Navidi, "Frequency Shift Keying Fundamentals", Hamilton Hallmark TSC News,Vol. 4, No. 4,

pp. 6-8, March 1996, http://www.hh.avnet.com