SMITH.DOC

This program gives you the possibility to plot the results of the measurements

you made with the noise bridge on the Smith-chart.

You can see the immediate results of the effect on VSWR with your inputs and

with the tuning possiblities.

To see the results on the graphic chart you need a VGA driver and hi-res moni-

tor, but even if you work with HERCULES,CGA or EGA you can look at the effect

on VSWR when you add for instance a serie inductor or a shunt tuned shunt net-

work or whatever you choose to adjust your antenna.

Example 1

Suppose you want to compensate the tower effect on 40-m slopers.

You measure Rs and Xs on 7, 7.1, 7.2 and 7.3 Mc with series bridge.

Suppose you made your measurements directly on the antenna borns.

You fill in "s" for serie and "4" for the number of measurements.

Then you fill in F, Rs and Xs for each frequency.

The results are ...

frequency         Rs          Xs          VSWR

7                 52        -110           6.5

7.1               57        -100           5.3

7.2               65         -90           4.3

7.3               70         -80           3.7

Choose add serie transmission line.

What is line impedance (ohm) ?     50

What is velocity factor ?         .67

What is length (in meters) ?      3.3

What is loss (in dB/100m) ?        2

frequency         Rs          Xs          VSWR

7                10.19      -25.27         6.2

7.1              12.02      -23.69         5.1

7.2              14.55      -22.75         4.2

7.3              16.87      -21.67         3.6

Choose add shunt transmission line.

What is line impedance (ohm) ?     50

What is velocity factor ?         .67

What is length (in meters) ?      2.54

Open (O) or shorted (S) stub ?     s

frequency         Rs          Xs          VSWR

7                71.58       -9.58         1.5

7.1              57.95       -6.56         1.2

7.2              50.13       -0.25         1.0

7.3              44.41        3.62         1.2

Example 2

Suppose you want your 2-meter whip beeing broadbanded.

You measure Rs and Xs on 110, 120, 130, 146 and 160 Mc with series bridge.

Suppose you made your measurements in the shack with a length of 25 meters

between your bridge and the antenna.The cable is RG213 with a loss of

8 dB/100m for this frequencies.

You fill in "s" for serie and "5" for the number of measurements.

Then you fill in F, Rs and Xs for each frequency.

The results are ...

frequency         Rs          Xs          VSWR

110               20          10           2.6

120               65         -30           1.8

130               40           0           1.3

146               30         -20           2.0

160               70          45           2.2

The results directly on the antenna borns can be found with adding a

negative transmission line length.

Choose add serie transmission line.

What is line impedance (ohm) ?  50

What is velocity factor ?      .67

What is length (in meters) ?   -25

What is loss (in dB/100m) ?      8

frequency         Rs          Xs          VSWR

110              53.47     -103.85         5.9

120              47.25      -48.53         2.6

130              69.20       -8.62         1.4

146              81.91       76.51         3.4

160             131.93       94.03         4.1

Choose add shunt inductor.

What is value of L (uH) ?   .06

frequency         Rs          Xs          VSWR

110              13.62       57.36         8.7

120              43.11       48.25         2.7

130              25.89       33.90         3.0

146              10.33       38.45         7.8

160              11.64       46.70         8.2

Choose add serie capacitor.

What is value of C (pF) ?    60

frequency         Rs          Xs          VSWR

110              13.62       33.25         5.4

120              43.11       26.14         1.8

130              25.89       12.46         2.1

146              10.33       20.28         5.7

160              11.64       30.12         5.9

Choose add shunt capacitor.

What is value of C (pF) ?    30

frequency         Rs          Xs          VSWR

110              77.28       36.76         2.1

120              38.56      -28.05         2.0

130              30.44       -8.69         1.7

146              37.41       21.81         1.8

160              88.41      -10.09         1.8

After you retuned your antenna with this inductor and capacitors you

should measure the following results in your shack.

Choose add serie transmission line.

What is line impedance (ohm) ?  50

What is velocity factor ?      .67

What is length (in meters) ?    25

What is loss (in dB/100m) ?      8

frequency         Rs          Xs          VSWR

110              36.48      -13.46         1.6

120              50.53      -21.12         1.5

130              59.92       15.65         1.4

146              59.44      -17.10         1.4

160              71.00        6.11         1.4

This program is written in TURBO-PASCAL for IBM.

This program is completely freeware and may be distribuated freely.

The autor is not responsible for any damage caused by the program.

Hope you'll enjoy it, remarks to ON1BWO @ ON6AR.