M O D E M C H K

~~~~~~~~~~~~~~~~~

by David Fraser ZL3AI @ ZL3VML.CHC.NZL.OC

E-mail: fraser@southlights.gen.nz

INTRODUCTION:

~~~~~~~~~~~~~

This program was developed to assist others to debug their BayCom style

modems and the wiring between them and the associated computer.  It may

also be used in a recreational manner by simply watching the display of

data from an unsquelched receiver and attempting to identify morse

characters and words if you have nothing better to do.

RECEIVE CHECKING:

~~~~~~~~~~~~~~~~~

It displays received data from the modem in a manner similar to an

oscilloscope, however it is not strictly accurate as the signal is only

sampled every 10 mS or so.  However the primary objective is to be able to

see if there is data coming from the modem or if the data line is stuck

high or low.

TRANSMIT CHECKING:

~~~~~~~~~~~~~~~~~~

The PTT line and transmitted data lines may be asserted and de-asserted

with F-keys.  Note that you can ONLY changed the tx data line IF the PTT

line is high, due the the kludgy way I power the modem.

TRANSMIT TIMERS:

~~~~~~~~~~~~~~~~

Be aware that there is generally a transmit timer in most reputable modem

circuits to ensure that your transmitter can't be keyed up for hours on end

due to misbehaviour elsewhere (i.e. your computer or its software, etc) so

if you are using MODEMCHK to tell your system to transmit for more than say

10 seconds, the modem may actually "time-out" on you.  If your modem does

not have a transmit timer, seriously consider adding one to avoid

Chernobyling your transmitter after jamming the local packet channel for

hours.  RS232 ports sometimes go to undefined states at undesired times.

POSSIBLE PROBLEM:

~~~~~~~~~~~~~~~~~

The modem is normally powered via 3 diodes - one from the RS232 TXD line,

one from the BayCom PTT line and one from the BayCom Tx data line.  In the

case of using MODEMCHK, I haven't figured out how to properly drive the

RS232 TXD line, however the program will ALWAYS have either the PTT line or

the BayCom Tx data line high, thus (I hope) ensuring the modem will be

adequately powered.  Some hardware designs may possibly fail to get enough

power, but tests so far on a variety of modems indicate that this has not

been a problem.  If in doubt, measure the DC supply voltage inside the

modem.  I would be grateful for reports as to whether or not MODEMCHK works

on various modems.

ADDITIONAL SOFTWARE:

~~~~~~~~~~~~~~~~~~~~

An excellent program worth looking at to improve reception is MDMTEST1.EXE

by Pawel Jalocha SP9VRC, which gives you a display of the distortion in

incoming data.  You tweak a preset pot in the modem for minimum distortion.

--------//--------