-----------------chop-with-axe----------------chop-with-axe---------------------

From: jfw@ksr.com (John F. Woods)

Subject: Telephone Caller ID specs:

Every now and then, someone asks again about Caller ID and how to

decode it.  It turns out that Radio Electronics published most of the

answer in the Hardware Hacker column in their August 1991 issue.  A

quick summary follows:

First, the relevant documents are:

1. NYNEX Catalog of Technical Information, #NIP-7400 (Free).

2. SPCS Customer Premises Equipment Data Interface,

#TR-TSY-0030, $25.

3. CLASS Feature: Calling Number Delivery, #FSD-02-1051, $30.

4. CLASS Feature: Calling Number Delivery Blocking,

#TR-TSY-000391, $33.

Document 2 is the most important, and can be ordered from

Bellcore at (800) 521-CORE; they take VISA.

The caller ID is transmitted as 1200 baud tones (Mark/1 is one cycle

of 1200 Hz, Space/0 is nearly two cycles of 2200Hz), 8 bits

asynchronous, one stop bit.

The data transmitted is: 30 bytes of 0x55 as a "channel seizure"

signal (when demodulated, looks like a 1/4 second 600Hz square wave);

150 milliseconds of all marks; a message-type word (one byte, value 0x04

indicates caller ID); a message length word (one byte, how many digits

in the calling number, does not include itself or the checksum); the

_ASCII_ digits of the phone number, least significant first; finally,

a checksum byte, consisting of the two's-complement of the 8-bit sum

of the message-type word, the message-length word, and the data.

This is the simplest form that the information will be delivered in,

you need the Bellcore specs if you want to get it right in all cases.

Sierra Semiconductor has two IC's which handle most of the analog

portion of caller-ID, the SC11211N and the SC11210 (which needs an

external oscillator and deletes some of the features); these chips

output a digital stream ready for digestion by a uC.

####

From: schuch@phx.mcd.mot.com (John Schuch)

Subject: Re: Caller ID info?

Phone gets first ring, duration 2 seconds

Delay 0.5 seconds after first ring

Send 30 bytes of '01010101', duration 250ms (channel seizure signal)

Send 150ms of marks (carrier signal)

Send an 8 bit "parameter word type"

00000100 = Caller ID

00001010 = message waiting indicator

10000001 = Test for caller ID

Send an 8 bit number representing the number of WORDS in the message

Now, the actual data:

The first two digits are the month with a leading zero for JAN to SEPt.

The next two digits are the day of the month w/leading zero for 1 to 9.

The next two digits are the hour in local military time.

The next two digits are the minutes in local time.

The remaining digits (message length -8) are the telephone number

of the calling party. The last field may include the 'P' or 'O'

for blocked or out-of-area calls.

The last 8 bits of the message are the checksum. The check sum is the

two's complement of the modulo 256 sum of the other words in the data

message. This sum does not include the channel seizure signal or the

carrier signal.

Total transmission duration = 718ms

The above description is as stated in the Motorola MC145447 Caller ID

Receiver data sheet, and in the Cermetek Application Note AN-107. I

have not yet seen the actual data but will have my CID > RS232 circuit

complete this weekend. I will then be able to shead a little more light

on the topic.

John

(PS- I have MC145447 chips for sale)

####

From: Someone I can't remember

Subject: CID specs

1. Caller Number Delivery (C.N.D)

s40 bit #2, 1: enable, 0:  disable (default)

Response message:

(1) Single message format

RING

MM-DD hh:mm Caller_ID

RING

example:

RING

04-28 12:30 035782439

RING

(1) Multiple message format

(a) Number and name are available

RING

TIME: MM-DD hh:mm

CALLER NUMBER: Caller_ID

CALLER NAME: Caller_Name

RING

example:

RING

TIME: 04-28 12:30

CALLER NUMBER: 035783942

CALLER NAME: ZyXEL

RING

(b) Number and name are not available

RING

TIME: MM-DD hh:mm

REASON FOR NO NUMBER: OUT-OF-AREA

REASON FOR NO NAME: PRIVACY

RING

example:

RING

TIME: 04-28 12:30

REASON FOR NO NUMBER: OUT-OF-AREA

REASON FOR NO NAME: PRIVACY

RING

MM: month

DD: date

hh: hour

mm: minute

Caller_ID: phone number of caller

Caller_Name: name of caller

2. Distinctive Ring

s40 bit #3: 1, accept ring of type 1

s40 bit #4: 1, accept ring of type 2

s40 bit #5: 1, accept ring of type 3

s40 bit #6: 1, accept ring of type 4

If all these bits are cleared (default), any ring of

duration longer than 100 ms will be accpted.

Type 1: 1.2/2 s on, 4 s off.

Type 2: 0.8 s on, 0.4 s off, 0.8 s on, 4 s off.

Type 3: 0.4 s on, 0.2 s off, 0.4 s on, 0.2 s off, 0.8 s on, 4 s off.

Type 4: 0.3 s on, 0.2 s off, 1 s on, 0.2 s off,  0.3 s on, 4 s off.

-----------------chop-with-axe----------------chop-with-axe---------------------