----------------------------------------------------------------------

This is the circuit for a simple FM wireless microphone, published in

the 1990 Radio Electronics "Electronics Experimenters Handbook", pp.

155-157.  It's simple and handy to have - pretty much the same as the

"Mr. Microphone" they sell (or used to) at Radio Shack.  Range of maybe

100 feet, on standard broadcast FM receivers.

___    ______________________________________________________________

\|/   ___     )       |       |      |		   |	    |	     |

|    ---     )       R      ___     R		   R	    |	   -----

|_||__| C10  ) L1    4      --- C3  5		   7	    |	    ---

||  |      )       |       |      |		   |	   ___ C6  ----- B1

C1  -------	      |       |      |		   |	   ---	    ---  9VDC

|_____	      |       |  C9  __R6__	   ----     |	     |

|     \	Q1    |       |      |	   |	   |   |    |	     |

|      \|______|_______|__|(__| Q2  |	   R   |    |	     |

___     /|      |       |  |(  \	   |  C5   8   |    |	     |

C2    ---    / BF199  |       |       \|___|__|(___|  _+_   |	     |

|____V	      R       |       /|      |(   |  ---   |	     o

|	      2      ___     / BC183C	   |   | C7 |	  \

|	      |  C4  ---    V		   |   |    |	   \  S1

R	      |       |     |_______  MIC / \  |    |	    \

1	      |       |     |	   |	  \_/  |    |	     O

|	      |       |     R	  _+_	   |   |    |	     |

|	      |       |     3	  ---  C8  |   |    |	     |

|	      |       |     |	   |	   |   |    |	     |

---------------------------------------------------------------

R1 = 100 ohms		C1 = 1.5 pf cer. disc		Q1 = BF199, NTE 229,

R2, R4 = 10K ohms	C2 = 100 pF, NPO		     ECG 229 or eq.

R3 = 1K ohms		C3, C4 = 330 pF NPO		Q2 = BC183C, NTE199

R5, R7 = 47K ohms	C5, C9 = .1 uF NPO		     ECG 123AP or eq.

R8 = 4.7K ohms		C6 = .001 uF NPO		L1 = approx. 1 uH

M1 = electret mike	C7 = 22 uF electrolytic 	S1 = SPST switch

B1 = 9V battery 	C8 = 6.8 uF electrolytic        R6 = 2.2 Megohms

C10 = 10 - 40 pF trimmer capacitor.

The coil L1 is so small that in the PC board version of this, it's just a coil

etched on the board with traces.  I think I breadboarded it with about four or

five turns of bell wire around a pencil and that made about the right value.

You tune the sucker with the adjustable cap C10.  If I remember, the

transistors are garden variety ones, though I think Q1 was reasonably high

frequency.  I was feeding in a signal other than an electret microphone.

This is neat to use with a cheapo FM walkman-type radio for cheap telemetry.

(We were actually going to build a device to transmit strain gauge signals

from someone's teeth over a whole day's time, and it was to be small enough

to be relatively unnoticed INSIDE THEIR MOUTH the whole time.  Never did

get the O.K. to proceed with that one.	Working for university research

gets pretty strange sometimes.)

Paul Marxhausen  University of Nebraska - Lincoln

Engineering Electronics Shop