### The project

This Arbitrary Waveform Generator (AWG) is based on a PIC microcontroller and a DAC. The waveform (25 words of 8 bit) and its frequency are stored into the microcontroller EEPROM via the serial port of a PC. Once programmed, the device can operate as a standalone unit. Waveforms in the frequency range from 78 Hz to 25000 Hz can be generated.

### Detailed description

Two modes of operation can be accessed via the button B1: programming mode (PM) if B1 is pressed during power-up, or wave generation (WG) otherwise. In PM the green led is on and the microcontroller, a Microchip PIC16F84, waits for 27 words of 8 bits to be transmitted via the serial port. The C program for DOS/Win95 included in this project can be used to this end. Interfacing between TTL (0, +5 V logic) and RS232 (-12 V, +12 V) is accomplished by a Maxim MAX232 interface driver. The first 25 words represent the waveform shape (0 corresponds to 0 V, 255 to +5 V output), the remaining two (F and HF) set the frequency according to the following table, in decreasing order of priority:

bit 0 of HF set	=>	f = 25000 Hz

bit 1 of HF set	=>	f = 20000 Hz

bit 2 of HF set	=>	f = 16666 Hz

bits 0, 1, 2 of HF clear 	=>	f = 20000 / (F + 1) Hz; where F must be >= 1.

The data are stored into the EEPROM of the microcontroller. Thanks to this, the AWG do not require reprogramming after a power down and can operate as a standalone unit. The C program awf.c reads the data from an ASCII file containing the 27 words in decimal format. For example, a file containing the following data: 0 10 20 … 230 240 1 0 will generate a sawtooth waveform at 10000 Hz frequency.

When programming completes or when the device is powered-up with B1 in its released position, the AWG enters the WG mode (red LED on) and the 25 words are fed into an 8-bit current-output AD0800 DAC whose output is converted into a voltage in the range 0 to +5V by a MAX406 rail-to-rail op-amp.

The device requires a dual supply (-5 V, 0, +5 V).

### Part list

| Part label | Part description | Quantity |
| --- | --- | --- |
| PIC16F84 | PIC 16F84 | 1 |
| MAX232 | MAX 232 | 1 |
| DAC0800 | #### AD 0800 | 1 |
| A1 | MAX 406 | 1 |
| D1 | Green LED | 1 |
| D2 | #### Red LED | 1 |
| R1, R2, R3, R4 | 5.6 K resistor ¼ W | 4 |
| R5 | 10 K resistor ¼ W | 1 |
| R6, R7 | 1 K resistor ¼ W | 2 |
| C1 | 22 nF poli. | 1 |
| C2, C3 | 68 nF poli. | 2 |
| C4, C5 | 33 pF ceramic | 2 |
| C7, C8, C9, C10, C11 | 1 uF elec. | 5 |
| B1 | Button | 1 |

C program awg.c source code

/*	awg.c									*/

/*	reads the waveform data from a file and outputs 	*/

/*	them to the serial port						*/

/*	© 2000 Mattoli-Piazza						*/

#include <stdlib.h>

#include <dos.h>

#include <stdio.h>

#include <conio.h>

#define PORT1 0x3F8

#define NPOINT 27

void main(int argc, char *argv[])

{

int cin, INTIME, trash, count = 0, waveform[NPOINT];

FILE *in;

if(argc != 2)

{

printf("Usage: awg <filename>\n");

exit(0);

}

if((in = fopen(argv[1], "r")) == NULL)

{

printf("Unable to open file %s\n", argv[1]);

exit(0);

}

outportb(PORT1+1,0x00); /* disable interrupts*/

outportb(PORT1+3,0x80); /* Set  DLAB ON*/

outportb(PORT1+0,0x60); /* Setting Baud Rate */

outportb(PORT1+1,0x00); /* Setting Baud Rate (MSB byte)*/

outportb(PORT1+3,0x03); /* serial port settings: 1200baud, NP, 1STb */

outportb(PORT1+2,0x0B); /* FIFO */

trash = inportb(PORT1);

while(!feof(in) && count < 27)

{

fscanf(in, "%d", &trash);

waveform[count] = trash;

outportb(PORT1, waveform[count]);

while(!(inportb(PORT1 + 5) & 1));

cin = inportb(PORT1);

delay(10);

if(cin != waveform[count])

{

printf("%d - %d %d\n", count, cin, waveform[count]);

printf("Error: check programming mode\n");

exit(0);

}

count ++;

}

}

Note: when programming the PIC, please set the fuses according to the following scheme:

Oscillator:        XT

Watchdog Timer:    OFF

Power Up Timer:    OFF

Code Protect:      as you like

PIC source code: main file

File name: awg.asm

;       awg.asm:	arbitrary waveform generator

;       Copyright 2000 Mattoli-Piazza

;

LIST    p=16F84

#include "P16F84.INC"

CBLOCK  0x0C

TEMP, TEMP1, AD

TXREG, RXREG, nE2PROM

ENDC

FREQ    EQU     0x39

HF      EQU     0x3A

;       wavefunction data: RAM addresses 0x20 to 0x3F

;                        : EEPROM   "    0x00 to 0x19

;       frequency        : address   FREQ

;       high freq.       :    "      HF

;       high-frequency flag:

;       HF:     bit 0 on -> freq0

;               bit 1 on -> freq1

;               bit 2 on -> freq2

;               bit 3 on -> freq3

;       PORT A details

TX      EQU     1       ;       PA0     Serial transmit

RX      EQU     0       ;       PA1     Serial receive

WL      EQU     2       ;       PA2     wave out LED

PL      EQU     3       ;       PA3     programming-mode LED

BT      EQU     4       ;       PA4     BUTTOM

;               0V -> enter programming mode

org     0x00

bcf     INTCON, GIE     ;       disable global interrupts

clrf    PORTA

bsf	PORTA,	TX

clrf    PORTB

bsf     STATUS, RP0             ;       bank 1

movlw   B'00010001'             ;       PORTA   in/out

movwf   TRISA                   ;

clrf    TRISB                   ;       PORTB   output

bcf     OPTION_REG,     T0CS    ;       disable TIMER 0

bcf     STATUS, RP0             ;       bank 0

bsf	PORTA,TX

btfss   PORTA,  BT      ;       if the button is pressed during

call    progr           ;       power-up enter programming mode

bcf     PORTA,  PL      ;       programming LED off

bsf     PORTA,  WL      ;       wave-out LED on

call    eewave          ;       copy the wavefunction data

;       from EEPROM to RAM

btfsc   HF,     0       ;       test the high-freq flag

goto    wavef0

btfsc   HF,     1

goto    wavef1

btfsc   HF,     2

goto    wavef2

goto    wavefn          ;       if HF is not set then call the

;       generic-freq. routine

;*********************************************************************

include	"sertxrx.asm"	;	serial tx-rx routine

include "mfreq.h"       ;       wave generation macros

;*********************************************************************

freq0   ;       macro   f  = 25000       Hz

freq1   ;       macro   f  = 20000       Hz

freq2   ;       macro   f  = 16666       Hz

freqn   ;       macro   fn = 20000/(n+1) Hz; n>1

;*********************************************************************

;       eewave: copy the wavefunction data from EEPROM to RAM

;

eewave  clrf    EEADR

eer1    movf    EEADR,   W

addlw   0x20            ;       0x20 is the starting RAM location

movwf   FSR             ;       for the wavefunction data

call    EEREAD

movwf   INDF

incf    EEADR,   F

movlw   0x1B            ;       have we read the 25+2 bytes?

subwf   EEADR,  W       ;

btfss   STATUS, Z       ;

goto    eer1

return

;*********************************************************************

;       progr: reads 25+2 data byte from the serial port

;

progr

movlw	0x00

movwf	nE2PROM		;nE2PEROM (number of bytes written in the EEPROM

bsf	PORTA,TX

bsf	PORTA,PL

bcf	PORTA,WL

poll	nop

BTFSC   PORTA,RX       ;test input

GOTO    poll           ;waits until RX=0

loop	BTFSC   PORTA,RX

goto	loop

call 	SERREAD		;

movf 	RXREG,W		;

movwf	EEDATA		;

movf	nE2PROM,W	;

movwf	EEADR		;

call	EEWRITE

call	EEREAD

movwf	TXREG		;

call	SERWRITE	;done writing?

bcf	STATUS,Z	;

incf	nE2PROM,F	;

movlw	0x1B		;

subwf	nE2PROM,W	;

btfss	STATUS,Z	;

goto	loop

return

;*********************************************************************

;       EEPROMW: write W in the EEPROM location

;                specified by EEADR

EEWRITE

bsf	STATUS,RP0	;

bsf	EECON1,WREN	;

movlw	0x55		;

movwf	EECON2		;

movlw	0xAA	;

movwf	EECON2		;

bsf	EECON1,WR	;

nop			;

loop1	btfss	EECON1,EEIF	;

goto	loop1		;

bcf	EECON1,WREN	;

bcf	STATUS,RP0	;

return

;*********************************************************************

;       EEPROMR: read the byte at the EEPROM address EEADR

;                and store it into W

EEREAD

bsf     STATUS, RP0     ;       bank 1

bsf	EECON1,	RD	;	EEPROM read

bcf     STATUS, RP0     ;       bank 0

movf	EEDATA,	W

return

END

PIC source code: serial routines

File name: sertxrx.asm

SERREAD

nop

nop

call	DELAY1

nop

nop

nop

nop

call 	DELAY

btfsc	PORTA,RX

bsf	RXREG,0

btfss	PORTA,RX

bcf	RXREG,0

call 	DELAY

btfsc	PORTA,RX

bsf	RXREG,1

btfss	PORTA,RX

bcf	RXREG,1

call 	DELAY

btfsc	PORTA,RX

bsf	RXREG,2

btfss	PORTA,RX

bcf	RXREG,2

call 	DELAY

btfsc	PORTA,RX

bsf	RXREG,3

btfss	PORTA,RX

bcf	RXREG,3

call 	DELAY

btfsc	PORTA,RX

bsf	RXREG,4

btfss	PORTA,RX

bcf	RXREG,4

call 	DELAY

btfsc	PORTA,RX

bsf	RXREG,5

btfss	PORTA,RX

bcf	RXREG,5

call 	DELAY

btfsc	PORTA,RX

bsf	RXREG,6

btfss	PORTA,RX

bcf	RXREG,6

call 	DELAY

btfsc	PORTA,RX

bsf	RXREG,7

btfss	PORTA,RX

bcf	RXREG,7

call 	DELAY

return

SERWRITE

bcf	PORTA,TX

nop

nop

nop

nop

call	DELAY

btfsc	TXREG,0

bsf	PORTA,TX

btfss	TXREG,0

bcf	PORTA,TX

call 	DELAY

btfsc	TXREG,1

bsf	PORTA,TX

btfss	TXREG,1

bcf	PORTA,TX

call 	DELAY

btfsc	TXREG,2

bsf	PORTA,TX

btfss	TXREG,2

bcf	PORTA,TX

call 	DELAY

btfsc	TXREG,3

bsf	PORTA,TX

btfss	TXREG,3

bcf	PORTA,TX

call 	DELAY

btfsc	TXREG,4

bsf	PORTA,TX

btfss	TXREG,4

bcf	PORTA,TX

call 	DELAY

btfsc	TXREG,5

bsf	PORTA,TX

btfss	TXREG,5

bcf	PORTA,TX

call 	DELAY

btfsc	TXREG,6

bsf	PORTA,TX

btfss	TXREG,6

bcf	PORTA,TX

call 	DELAY

btfsc	TXREG,7

bsf	PORTA,TX

btfss	TXREG,7

bcf	PORTA,TX

call 	DELAY

nop

nop

bsf	PORTA,TX

nop

nop

call	DELAY

bsf	PORTA,TX

nop

nop

call	DELAY

return

DELAY                           ;delay routine for serial transmission

nop                     ;the delay corresponds to the

nop                     ;transission time of a single bit at

nop                     ;1200 baud with a 10 MHz-clock processor

nop

movlw	.207

movwf	TEMP

de1	nop

nop

nop

nop

nop

nop

nop

decfsz	TEMP,	F

goto	de1

return

DELAY1                          ;delay routine for serial transmission

nop                     ;the delay corresponds to HALF of the

nop                     ;transission time of a single bit at

nop                     ;1200 baud with a 10 MHz-clock processor

nop

movlw	.103

movwf	TEMP

de2	nop

nop

nop

nop

nop

nop

nop

decfsz	TEMP,	F

goto	de2

return

PIC source code: macros

File name: mfreq.h

;*********************************************************************

;       macro freq0: 25000 Hz function generation

;

freq0   macro

local a=0x20

wavef0

while a<0x38

movf	a,	W

movwf	PORTB

nop

nop

a+=1

endw

movf	0x38,	W

movwf	PORTB

goto	wavef0

endm

;*********************************************************************

;       macro freq1: 20000 Hz function generation

;

freq1	macro

local a=0x20

wavef1

while a<0x38

movf	a,	W

movwf	PORTB

goto	f1#v(a)

f1#v(a)	nop

a+=1

endw

movf	0x38,	W

movwf	PORTB

nop

goto	wavef1

endm

;*********************************************************************

;       macro freq2: 16666 Hz function generation

;

freq2	macro

local a=0x20

wavef2

while a<0x38

movf	a,	W

movwf	PORTB

goto	f2#v(a)

f2#v(a)	nop

nop

a+=1

endw

movf	0x38,	W

movwf	PORTB

goto	f2r

f2r	goto	wavef2

endm

;*********************************************************************

;       macro freqn: generic frequency generation

;

freqn	macro

local a=0x20

wavefn

while a<0x38

movf	a,	W

movwf	PORTB

goto	fn#v(a)

fn#v(a)	movf	FREQ,	W

movwf	TEMP

fnc#v(a)

nop

nop

decfsz	TEMP,	F

goto	fnc#v(a)

a+=1

endw

movf	0x38,	W

movwf	PORTB

movf	FREQ,	W

movwf	TEMP

fnc#v(a)

nop

nop

decfsz	TEMP,	F

goto	fnc#v(a)

goto	wavefn

endm