SYSTEST v2.01

This utility reports CPU type and speed, and measures memory,

video, and ISA bus access timing.  Also shows DOS program start

address, memory end, BIOS date, various CPU flags, Advanced

Power Management status if present, and SMARTDRV version and

status.

Here is a typical output example:

����������������������������������������������������������������������������

SYSTEST version 2.01

Copyright (C) 1999 by Interstellar Research  -  http://www.daqarta.com

DOS End    Prog CS     IRQs Enabled      CPU Flags     CPU Status     BIOS

9FC0h      0D33h    .EDC..9..6...210    0000:7046h       0010h     04/19/90

Video: VGA, mode 03h.  Mode 10h byte read:   498, write:   210 nsec

CPU:   50.0 MHz 486.

Genuine Intel Original OEM 486, Model 3 (DX2), Step 5.

Instruction times (nsec):

LPT 1    INT MASK     DMA     TIMER     STRINGw    WORD

Read:   1408      1408      1408      1408        99        20     (Base)

Write:   1495      1496      1496      1495        99        19       180

ADVANCED POWER MANAGEMENT --- APM version 01.01 supports:

16-bit   32-bit   CPU Idle    ------- BIOS --------

YES      YES        NO       DISABLED   DISENGAGED

SMARTDRV version 05.01 installed.  Only READ Cache enabled for Drive C:

Use SYSTEST ? for more information and options.

����������������������������������������������������������������������������

What it means:

DOS End is the start of memory segment just past that which

normal real-mode DOS can access.  This is limited to A000h, the

start of video memory, but may often (as in the example above)

be a little less.

Prog CS is the start of the actual SYSTEST code segment, so the

amount of DOS memory available for programs is the difference

between these two.  In this example, that would be 9FC0h -

0D33h = 928Dh paragraphs (at 16 bytes each), or 928D0h bytes

total.  Converting from hexadecimal, that's 600272 bytes.

IRQs Enabled shows the number of any IRQ (Interrupt ReQuest)

that is presently enabled.  For a compact display, those above

9 are shown in hexadecimal, so in this example IRQs 14, 13, 12,

9, 6, 2, 1, and 0 are enabled.  Note that these are only those

that are currently enabled, not necessarily in active use.

Also note that other IRQs may be reserved for specific uses,

but will not be enabled until needed.

CPU Flags are the contents of the EFLAGS:FLAGS registers,

expressed as a hexadecimal value for compactness.

Bits 31-19   (Reserved on 486 CPU)

18      Alignment Check

17      Virtual 8086 Mode

16      Debug Resume

15      (Reserved)

14      Nested Task

13-12   I/O Privilege Level

11      Overflow Flag

10      Direction Flag (String operations)

9      Interrupt Flag

8      Trap Flag

7      Sign Flag

6      Zero Flag

5      (Reserved)

4      Auxiliary Flag

3      (Reserved)

2      Parity Flag

1      (Reserved)

0      Carry Flag

CPU Status is the contents of the Machine Status Word (MSW)

on 286 and higher CPUs, expressed in hexadecimal.

Bit 31      Paging (386+)

30      Cache Disable (486+)

29      Not Write-through (486+)

28-19      (Reserved)

18      Alignment Mask (486+)

17      (Reserved)

16      Write Protect (486+)

15-6      (Reserved)

5      Numeric Error (486+)

4      Extension Type

3      Task Switched

2      Emulate Processor Extension

1      Monitor Processor Extension

0      Protected Mode Enable

BIOS is the BIOS release date, found at F000:FFF5h in all

systems.  There is no standard location for the BIOS

manufacturer, though it usually appears in a copyright message

somewhere in the region right after F000:0000.  You can see

that via the DEBUG D (Dump) command:

DEBUG

D F000:0000  (Dump memory at this location)

...          (Hit D additional times to see more)

Q            (Quit DEBUG)

Video: shows the type of video adapter:

MDA     Monochrome Display Adapter

HGC     Hercules Graphics Card

CGA     Color Graphic Adapter

EGA     Enhanced Graphics Adapter

VGA     Video Graphics Array

The VGA label is also applied to anything higher, such as SVGA

or XGA, as well as the MCGA (Multi-Color Graphics Array) built

into PS/2 systems.

For VGA systems, many older cards had only 8-bit ISA connectors,

and even some early cards with 16-bit connectors were only

capable of 8-bit video accesses.  These cards will show as

VGA (8 bit)

The current video mode (at the DOS command line prompt) is

typically 03h.  MDA or HGC cards will be 07h.

The times to read and write a byte of data from/to the screen

are shown for 640x350 graphics mode 10h on EGA cards and above,

or else normal text mode for other cards. (Mode 10h is used by

Daqarta for speed and widest compatibility with graphics

cards.)  These times actually include the time to advance a

pointer to the next screen byte location, since they use

repeated string operations (LODSB and STOSB).

CPU speed is shown to the nearest 0.1 MHz, but note that this

is a tricky measurement that may vary a few MHz between

attempts.

x86 CPU family should be correct for all Intel CPUs, including

the rare 8086 (used on some PS/2s) and the even rarer 80186 and

80188 (untested... may have never actually been used in any

PC).  The old NEC V20 is properly detected, as well as any

modern chip that supports the CPUID instruction.

The 386SX and DX can only be distinguished in real-mode DOS,

not in protected mode.  You will need to either boot DOS

directly, or use the Windows 9x 'Restart in MS-DOS Mode'

option from the Shut Down menu.  If SYSTEST detects a 386

in protected mode, it reports:

(No SX/DX test in protected mode.)

For CPUs that do support the CPUID instruction, the next line

shows the results.  SYSTEST shows the Model and Step numbers,

and translates those that it understands...in the above

example, it recognized a (DX2).

Instruction times are shown for various operations.  If

the system is running in protected mode, this header will say

PROTECTED MODE Instruction times

to make this clear, since that can have a drastic effect upon

the I/O port access times.

Protected mode "protects" one application from another by

strictly limiting I/O access, and must consult a table of

allowable ports on each attempt.  It may decide to emulate the

operation instead of actually doing what is requested.  The

system TIMER, in particular, may be emulated to avoid

disturbing the time-of-day and resource allocation operations.

To make these timing measurements, SYSTEST first measures the

time to execute a base loop containing only 2 NOP instructions,

then times each test loop that also includes the instruction to

be measured.  The difference between base and test times is

then converted to nsecs per single loop pass and reported.

The (Base) time is also shown at the far right, for reference.

Both Read: and Write: times are measured for

each type of operation.  Note that for the I/O ports

(LPT 1, INT MASK, DMA, and TIMER) these are all BYTE

operations, whereas the other two are WORDs.

For the LPT 1 measurements, SYSTEST first establishes

that a valid port is present.  If not, it shows values of 0.

The Read value is for the Status port and the Write value is

for the Data port, as in normal usage.  For example, if the

LPT 1 Data port is at 0378h, the Status address would be 0379h.

The INT MASK is at 21h, used to enable/disable IRQs 0-7.

The value obtained by reading this port is the value that is

used for the write test, so that nothing is changed.

DMA timing is not for an actual DMA transfer operation,

since SYSTEST must be able to work on any system without

actually knowing what hardware is present or safe to use.  The

Read value is the time to read the DMA channel 1 pointer, which

is used by applications (like Daqarta) that need to monitor DMA

transfer progress between DMA-generated interrupts.

The DMA Write value is the time to set the DMA byte-reset

latch.  This port (0Ch) is normally used before sending word

data to the DMA controller, which can only accept one byte at a

time.  Writing anything to this port just resets the controller

to expect the low byte first, the normal order.

The TIMER Read value is the time to read the current

system timer 0 (time-of-day) at port 40h.  Since this timer

is normally set to count down 16-bit values, a read must be

done in two single-byte steps.  But since the timer is still

running, the count value may change between reading the first

and second byte.  That's not important here, since SYSTEST

ignores this timer value anyway.  (It uses timer 2 for

instruction timing.)

But for an accurate reading, the timer provides an instruction

to latch the current count value, so that the separate reads

can be done at leisure while the timer continues to count.

The timer 0 latch command is what is timed for the Write value

shown here... even though the latched value is never read back.

In normal use, a program would give a latch command followed

by two read commands, along with the necessary register setups

and shuffling to combine the two bytes into a whole word.

STRINGw shows the times to read / write one word of data

from / to memory as part of a repeated string, using REP LODSW

for the Read and REP STOSW for the Write.  A string command

uses a separate register to point to the start of the memory

block, then advances that pointer after each word is read or

written.

The REP STOSW command is commonly used to fill a block of

memory with the same value (from the AX register), such as to

clear an array.  If the pointer register is set for the

video memory area, this same command can be used to clear or

color a portion of the display image, though the byte-sized

STOSB version is often more useful.  (It was used for the

earlier video write measurement.)

REP LODSW, on the other hand, is completely nonsensical in

normal use, since it repeatedly reads a word into the AX

register and then ovrewrites it with subsequent words.  But

since the reported times are for each word, not the entire

string, it's a meaningful measure of the way a single LODSW

instruction is normally used, such as in processing values from

an array.

WORD shows the time to perform a simple read or write

to a memory location, using MOV AX,[SI] for the read, and

MOV [SI],AX for the write.

ADVANCED POWER MANAGEMENT reports if this is present,

and shows the version if so.  If found, the next lines show

what features are present.

o   16-bit protected mode interface support.

o   32-bit protected mode interface support.

o   CPU idle call reduces processor speed.

o   BIOS Power Management:

o   Enabled / Disabled

o   Engaged / Disengaged

If SMARTDRV is installed it is shown here, along with

the version number.  (Only versions 4.00 and later are shown.)

The read and write cache status is also shown for drive C:

only.

The final line of the normal display indicates that you can use

SYSTEST ?

for more information and options.  If you do that, you will see:

���������������������������������������������������������������������������

o  Instruction times for I/O ports are for BYTE reads and writes.

o  INT MASK is at 21h.  DMA reads channel 1 pointer, writes byte-reset.

o  TIMER reads timer 0 (time-of-day), writes timer 0 latch command.

o  Protected Mode (Windows or EMM386) may result in MUCH longer times.

o  STRINGw times are for LODSW and STOSW aligned on DWORD boundary.

o  The (Base) time is for a DWORD-aligned loop containing only 2 NOPs.

o  Test times are for 2 NOPs plus the test instruction, less (Base) time.

ADVANCED USERS ONLY:

Reads and writes of LPT 1 Status and Data ports may be replaced by

specifying another port address (hex) on the command line, as in:

SYSTEST 310

Specify WORD reads and writes at that address via:

SYSTEST w310

The write test of the specified port uses the data read from it.

Output may be redirected to a file with:

SYSTEST > filename.ext.

���������������������������������������������������������������������������

The first part of this is just a summary of the intruction timing,

as previously discussed.

The ADVANCED USERS ONLY section is important:  It allows

you to totally screw up your system if you don't know what you

are doing.  By supplying a port address, in hexadecimal only,

you can tell SYSTEST to skip the LPT 1 tests and use that port

instead.  The specified port will be read and the data saved,

then the port will be read repeatedly to get the Read timing.

The data is then written repeatedly to get the Write timing.

NOTE that just because SYSTEST writes back the same data as

read does NOT mean that nothing is changed:  Some devices like

the DMA and video controllers use a 2-step sequence to write

word data as separate bytes.  Many devices have completely

different meanings for the data that is read and written at the

same address, such as Status when reading and Command when

writing.  So DO NOT mess around here if you don't know what

you're doing!

The port address you enter will replace the 'LPT 1' header over

the output Read and Write times, and will be shown with a 'b.'

prefix, as in 'b.0310h'.  The prefix indicates a 'byte' data

size, which is typical for most I/O ports on a system.

By putting a 'w' before the specified address, you can tell

SYSTEST to use WORD reads and writes at that address.  This is

useful for testing the speed of devices on the 16-bit ISA bus

that support word transfers (not all do).  Note that your

system may add different numbers of wait states for 8-bit and

16-bit devices, so this allows you to see that.

(Note:  SYSTEST Version 2.00 still showed the 'b.' prefix in

the header when 'w' was specified, even though it did indeed

use word reads and writes.  The header is properly updated in

this Version 2.01.)

If you use the 'w' with an 8-bit port, the net effect is simply

to perform back-to-back operations on the specified address and

the one after it.  The data read from each port will be written

back to the proper port during the write test.

Finally, you can use standard DOS redirection to send the output

of SYSTEST to a file.  This is useful to gather information

from different systems for later comparisons.  Note that

redirection will overwrite any file of the specified name

without prompting you, so use caution when selecting a name.

�������������������������������������������������������������

Please check out our DAQARTA Website:

D A Q A R T A

Data AcQuisition And Real-Time Analysis

Shareware from Interstellar Research

http://www.daqarta.com

�������������������������������������������������������������

Questions?  Comments?  Contact us!

We respond to ALL inquiries, typically within 24 hrs.

INTERSTELLAR RESEARCH: 20 Years of Innovative Instrumentation

Copyright 1999 by Interstellar Research

tech@daqarta.com