�������������������������������������������

DTMF Real-Time Decoder --  386 DEMO Version

Shima-nized Advanced Utilities - J.M. Shima

Copyright (c) 1994-95 by James Shima

All Rights Reserved

�������������������������������������������

INTRODUCTION

������������

* This program is capable of decoding DTMF tones from a Soundblaster

Version 1.0 - 2.0, Soundblaster Pro, or TRUE Soundblaster compatible

(16-bit or 8-bit) sound card in real-time.  Soundblaster 16 support

is assumed but untested at this point.

* This 386 version is geared for slower machines that cannot perform

real-time processing with the 486 version.  HOWEVER, due to the

streamlining of the algorithm, results MAY NOT be as accurate as

the 486 version and can be susceptible to more decoding errors than

the 486 version.

* This program attempts to use your coprocessor for faster floating

point speed.  If you do not have a coprocessor, use the DTMF386D.EXE

executable.  Otherwise, use DTMF387D.EXE for coprocessor support.

* The Soundblaster or compatible sound card MUST be connected to port

address 220h and using DMA 1 (the factory settings).

* This program accepts COMMAND-LINE ARGUMENTS that are passed to the

DTMF decoder.

* For a list of the arguments, type DTMF386D at the DOS prompt.

* The DEMO only decodes a maximum of 4 numbers.  Interested

persons can receive the REGISTERED version with no limitations.

SOFTWARE LICENSE

����������������

The 386 DEMO version of the DTMF real-time decoder is shareware, and is free

for personal use only.  This is true as long as the DEMO is

not alterted in any fashion.  Registered users will receive the REGISTERED

version of the DTMF decoder and will receive documentation stating such.

The author offers no warranties on this program.  This program is provided

as is.  Use this program at your own risk.  I assume no responsibility in your

actions while you are using this program.  The author is not liable for any

damage caused to your computer while you are using this program.  Using

the program is your compliance to this statement and the license.

COMMAND-LINE ARGUMENT SCREEN

����������������������������

The program argument screen should look as follows:

Real-Time Dual-Tone Multi-Frequency Decoder, 386 DEMO Version

Shima-nized Advanced Utilities (c) 1994-95 - J.M. Shima

USAGE: DTMF386D  <sample_rate>  switches:[ <-t>number ]

Notes:  <sample_rate> = rate of incoming samples in Hz. The range

is 4000 < fs < 12000 for DTMF decoding.

The RECOMMENDED value is 4000 Hz for real time

decoding.

** Sound card must be Soundblaster 1.0 - 2.0, Pro, or Soundblaster

** compatible connected to port 220h using DMA 1 (The factory settings).

Switches: -t = Defines the S/N threshold value (in dB) that DTMF tones

must surpass for proper detection.

The threshold S/N value can be any real, non-negative number.

DEFAULT value is 20 dB.

EXPLANATION OF COMMAND-LINE ARGUMENTS

�������������������������������������

<sample_rate>   -->  Input sample rate.  Must be in the range of 4000 to

12000 Hz.  For DTMF detection, it is HIGHLY

RECOMMENDED THAT YOU USE 4000 Hz for

real-time processing.

Switches:

��������

-t<number>    -->  The threshold that the DTMF tones must exceed over the

noise floor for detection (in dB).  Typically, strong

DTMF tones will be 20-40 dB stronger than the noise

in the signal.  However, noisy signals lower this

signal-to-noise (S/N) ratio.  The threshold parameter

allows the user to set the S/N threshold for detection

of noisier or corrupted DTMF tones.

* NOTE:  If nothing is entered, then the DEFAULT S/N

threshold value is 20dB, which is a figure

of merit for normal tones.

One must realize that dB is a logarithmic scale

of signal gain through the equation:

dB = 20 * log(signal gain).

Thus, a large signal gain represents a small

increase in the dB number (i.e. 40dB is a 100x

signal gain, while 20dB is a 10x signal gain).

EXAMPLES OF RUNNING DTMF386D

����������������������������

* Invoke the program by typing the executable name.

The program ENDS when ANY key is hit during decoding!

DTMF386D 4000            -->   Input sample rate is 4000 Hz.

Input S/N threshold defaults to 20dB.

DTMF386D 4000 -t25       -->   Input sample rate is 4000 Hz.  The input

S/N threshold is set to 25dB.

DTMF386D 4000 -t15       -->   Input sample rate is 4000 Hz.  The S/N

threshold is set to 15dB.

IMPORTANT NOTES

���������������

Since this is a real-time piece of software, certain rules must be followed

for proper use.

Sampling rate:

Increasing the sampling rate means that samples are ready

faster.  Thus, the computer has less time to do what

is has to before it has to fetch more samples.  So,

one must make the sample rate as low as it can go

and still represent the maximum frequency in the signal.

For DTMF tones, this minimum sampling rate is 4000 Hz,

(due to Nyquist theorem).  IT BEHOOVES YOU to use this rate.

Sampling at a rate higher than necessary gains you

nothing, but INCREASES your computational overhead. For real-

time operation, make your sampling rate as low as you can

without violating Nyquist theorem (2x the maximum freq.

in the signal).  So, for all practical purposes, use

4000 Hz as your sampling rate.

Real-time processing:

If your system cannot handle real-time processing, that

means data will be lost.  While your system is processing

the old data, it doesn't have time to retrieve the new

data coming in.  This new data is discarded and the

computer picks up fresh data when it's done with its

current task.  Thus, results are inconsistent when real-time

processing cannot be done.  The probability of detecting

a given tone depends on where your computer is in its

processing stage.  You can see that its nearly impossible

to debug whether or not real-time processing is happening

or not.  On top of this, your CPU takes care of many other

housekeeping tasks.  Therefore, I have deemed "real-time"

on the fact that a DTMF tone does not go undetected as given

in the DTMF standards.  Closely spaced tones will be detected

in real-time if your system can run the code in real time.

Noisy DTMF tones are also error ridden since the processing

window is so small no noise immunity algorithm can be

implemented.  Note that non-pure tones may not be detected

properly!

INCLUDED FILES

��������������

DTMF386D.EXE   -->  DTMF real-time decoder, 386 demo version.

DTMF387D.EXE   -->  386 demo version with 80387 coprocessor support.

DTMF386D.DOC   -->  This file.

REGISTER.DOC   -->  Registration form.

REGISTRATION

������������

** For registration, please fill out REGISTER.DOC and see the DEMO screen.

** Specify media dispersement.

The REGISTERED version of this program allows unlimited decoding of numbers,

and offers other features including:

1) Automatic logging of the resulting decoded numbers to a log file.

2) REGISTERED users will also receive a DTMF generator program.  This is

a DTMF tone generator which allows you to create any DTMF sequence

you wish.  The program allows you to specify a DTMF number sequence,

the sampling rate, and how long you want the pulses to be (in samples).

TESTING

�������

I have thoroughly tested this code and I am not responsible for any mishaps

that may happen when run on your computer.

The code was tested on an IBM compatible 25MHz 486DX Cyrix upgrade PC

with a Soundblaster Version 1.5 sound card running DOS 5.0.  Using

test vector data, it was able to run in real-time mode with a

4000 Hz sample rate and decoded numbers with 85-95% accuracy.

Noisy tones gave inconsistent results.  Longer duration tones were

always decoded, while the errors were present in the short burst dials!

I have not tested the code on a Soundblaster 16-bit sound card, but the

tested Soundblaster compatible 16-bit card did behave properly.

Every action has been taken to ensure an error-free program.

Note, incompatibilities may stem from hardware deviations.

For optimal performance, DO NOT launch this program while running under

Microsoft Windows.

For more information,

EMAIL:  shima@eel.ufl.edu

US MAIL:        J.M. Shima

3800 SW 34th St.  Apt. J-80

Gainesville, FL  32608

TRADEMARKS

����������

Soundblaster is a trademark of Creative Lab's Inc.

Microsoft Windows and DOS is a trademark of Microsoft.