With this circuit you can modify how your voice sounds by changing the pitch of your voice. This circuit can be connected to

a phone and with a duplicate circuit on the end of the phone line, you can have a scrambled voice communication. The way the circuit works is as follows:

If we cut the circuit in half at the T2 transformer and include the LM324 on the left side, you will see that the LM324 portion of

the circuit is a tone oscillator which shifts the frequency of all input signals to a new higher frequency.

When the voice and the

tone oscillator mix frequencies the voice is not recognized. The voice signal is then

inputted to the second stage which again shifts the voice signal again.

I recommend that the first stage be tuned to a frequency that is 100hz lower then the second stage.