Designing a Simple 2-way Speaker using Soundeasy

# John Kreskovsky

## March 2001

### Introduction

This documents briefly presents an outline of the design of a simple sealed box 2-way speaker system based on the Vifa P13WH-00-08 woofer and D25AG-35-06 tweeter. The P13 is a very nice, relatively inexpensive driver. On the other hand the D25AG is an aluminum dome tweeter that sound like most metal dome tweeters: someone beating on a garbage pail. Just a little humor there; however, I personally find this tweeter extremely distasteful and unmusical. But that’s not the intent of this document. How to design a speaker with Soundeasy is. So let’s get to it,

#### Box Design

To start I measured the T/S parameters of the P13 using my IMP. The woofers are relatively new, so they aren’t broken in. Thus the T/S parameters probably won’t reflect the actual driver parameters. But this is only intended to demonstrate the required steps in the design, not a true, time-tested design. The data showed Fs = 70.5 Hz, Re = 5.8, Le=0.67 mH, Qms=1.40, Qes=0.574, Qts=0.407, and Vas = 7.18 L. These numbers don’t agree with the Vifa specs, as expected. Never the less I used them as the basis of the box design. I had a small box around that was 1.79 L in volume. With the driver’s Vas, this would yield an alpha of 3.99 which should yield a Qtc=0.9096, a Fc of 157.6 Hz and an F3 of about 130 Hz. This seemed acceptable for this study. The box outside dimensions were 5” deep, by 6.5” high and wide. There was no baffle on the box (the front side was open). I also had a 6.5” wide by 11” high baffle that had the proper cutout for the woofer in it. The woofer hole was center 3.25” from the side and bottom of the baffle. I cut an additional hole in the baffle to mount the tweeter and I was set. I screwed the baffle to the box using weather stripping as a seal.  I mounted the woofer in the box and measured the impedance curve to check the alignment. The measurement showed Qtc = 0.901 and Fc = 154 Hz, close enough. I next mounted the tweeter on the baffle and I was ready to start taking the necessary SPL and impedance measurements.

### SPL and Impedance Measurements

I didn’t want to deal with tweeter diffraction effects as they would only serve to complicated the crossover design process if I were to try to correct the on axis response of the system for them. Thus I choose to design the system based on measurements taken horizontally off axis by 15 degrees. I positioned the microphone 0.5 meters from the speaker with the mic positioned vertically at the same height of the woofer surround. I started by taking the tweeter SPL data shown below

As can be seen, the response is fairly smooth and shows a smooth roll off. The data reflects 1/6th octave smoothing. Next I measured the woofer SPL, shown below.

The same smoothing was used. In both cases the phase data includes the excess phase due to the path length delay to the microphone. One final SPL measurement was taken with both the woofer and tweeter driven at the same time. That result is below. I show only the amplitude response because the phase is not relevant.

##### Next I measured the impedance for both drivers. While I do not show this data, the SPL and impedance files are available for download at the Soundeasy e-Group site.

### Importing the SPL and Impedance data

##### The next step in the design process is to import the SPL and impedance data into Soundeasy. I did this following the method outlined in my article “How to Avoid the Pitfalls of Importing SPL data Into Soundeasy”. For the tweeter, the final result was to import the data with a delay of 1.47353 msec using 2nd order slope correction for both LP and HP. I also used –96 dB for “scale” and 90dB for “efficiency” to correctly scale down the SPL data by 6dB to represent the 1m level, although this is not necessary.  Also note that a “scale” factor need only be input when the SPL data is in absolute rather than relative form. The result is shown below.

##### The tweeter impedance was than imported the transfer functions computed and the tweeter file saved. I then imported the woofer SPL data and found the needed delay was 1.49412 msec when using 2nd order HP and 6th order LP slope corrections. This corresponded to an additional 7mm in the path length to the woofer. I also scaled the SPL data level as I did with the tweeter and edited the data below 200 Hz to approximate the actual woofer response. This aided in matching the driver phase at lower frequencies. The final result is shown below.

I then imported the woofer impedance data and saved the woofer file. Finally I imported the SPL data when the woofer and tweeter were measured at the same time. There is no need to worry about phase here, but the SPL level must be scaled as with the woofer and the tweeter. The result is below. Note I edited the response below 200 Hz as with the woofer. This file was also saved as a driver file.

### Crossover Design

The next step is to begin the crossover design. This is done using the crossover module. You should open this module now, but there is no need to close the box module. However, prior to opening the crossover module, either minimize the box module window, or click the center, “resize” button. If you click the resize button the Task bar at the bottom of your screen should be visible. What ever, you can figure it out.

There are several ways to approach the crossover design. One is to open the woofer or tweeter file and set up an appropriate filter for the specific driver, choose the desired target response and optimize the filter. Once this is completed for both drivers, the filter must be combined in a single schematic and saved as a project file for use in the sum_plot module. While this procedure is fine, and perhaps simpler for the beginner, it means you have to “draw” out the crossover twice. A more direct way is to start by going to the file menu and choosing “New Project”. When the project window opens, enter the complete path name for the woofer and tweeter driver files you saved in the box module. (If you are like me and have trouble remembering the full path name, cancel the project, go to the file menu and open the woofer file. The full path name to the open driver will now appear at the top of the Soundeasy window. Then, open a new project and type in the full path to the file as displayed. Hopefully, the second driver is in the same directory, so you can copy and paste the woofer path to the tweeter slot in the project box and just add the tweeter file name.) Ok, assuming this has been done you should have a blank crossover screen in front of you. Now, go back to the file menu and choose “Save Project”. I do this just so I don’t have to go back and enter the driver file names in the project box should something crash the program.

Now we are ready to design the crossover. What I do here is to draw out the topology of the crossover I intend to use. In this case, I want to design something approximating a 4th order L/R crossover. Since I have some experience with these drivers, I choose a 2nd order topology with Zobel for the woofer and a 3rd order topology with Zobel and an input resistor for the tweeter. (If I were less experienced I might try a 4th order topology on the woofer and the tweeter. I’d also use R/C series shunts in the woofer topology instead of just caps, and keep the tweeter Zobel and input resistor. Form there, I work through the design, and then go back and see if the same result could be accomplished with fewer components. This is one area where looking at individual filters can be useful.)

The topology I choose along with my initial estimate for component values is shown below.

You should note that I have assigned the driver types to the symbols by clicking on them, just as you do for a component. I have also assigned values to all the components. How did I come up with the values? Experience! They are probably not that good, but they are realistic and shouldn’t give any truly wired response characteristics. For example, in the woofer x-o it is better for the inductor to be too big and the shunt caps to be too small. The same thing applied to the tweeter network. This results in “low Q” filters that won’t have resonance peaks. Similarly, I always use about 4 to 6 ohms for a tweeter Zobel and start with about 10 ohms in the woofer Zobel. Again, at this point I save the project file (just over write the first one).  The next thing I do is just look at the response of both drivers. Go to the tool menu and click on frequency domain. When the plot window opens click on “plot”, “Amplitude”. Enter the driver types in the appropriate slot, 1 for the woofer and 4 for the tweeter. Then enter “0” for the “from” connected to node for the woofer and click plot now. Then repeat the process but replace the “0” with a * for the woofer and select “0” for the “from” node for the tweeter. If you have done everything correctly you should see a plot that looks like the one below.

If you have gotten this far, good. Now once again click on the plot button and enter “*” for the tweeter from node, but now enter 7 for the woofer “from” node and click plot now. You should see a plot that looks like the one below.

Looking back at the previous figure where both the tweeter and woofer SPL are shown we see a fairly wide region of overlap centered about 3 kHz. This, combined with the known off axis response of the woofer from Vifa data sheets, indicates that the 3 kHz region is probably a good choice for the crossover. Also, looking at the figure at the figure just above, it would appear that a target SPL level of 87 dB might be reasonable. Note, this is not a very scientific way to determine the target sensitivity, but in this case I consider it reasonable. If you try to reduce the system sensitivity to increase the bass extension, you will have to deal with compensation of the woofer impedance peak at 160 Hz.

At this point we are ready to begin the crossover optimization for the woofer. Be sure that the tweeter nodes are all specified as * in the plot box and that the correct node (#7) is specified in the woofer “from” slot. Now, go to the Tools menu and select filter/crossover optimization. Lock components 0 through 6 in the optimization window as they are in the tweeter network. Then click the optimize button and select a 4th order L/R low pass filter. Set the low pass frequency to 3000 Hz. Also set the optimization range at 200 to 4300 Hz. Finally set the attenuation to –3 dB and then click “optimize”. Allow the code to run and when it is finished it should have a response curve that looks like the one shown below.

Now, click “use optimized” and return to the CAD schematic editor screen using the “tools” menu. I recommend that you save the project at this point using the “save project” file command. I recommend that you name it “intermediate” for the sake of reference.

Now open the plot window again and click “plot”, “amplitude”. Change the tweeter from node to #4 and replace the #7 woofer node with *. Then click plot now. You should see a plot of the filtered tweeter SPL. If not, check what you have entered in the plot boxes. Assuming everything is ok, open the optimization window again. Now, unlock all the tweeter network components EXCEPT the resistor in the Zobel. Also lock all the woofer components. When done, components 0-5 should be the only ones unlocked. Click the optimization button and select a 4th order HP L/R filter. Set the crossover frequency to 3000 Hz, the attenuation to –3dB, and set the optimization range to 1500 to 20000 Hz. When completed the response should look like this

Make sure to click “use optimized” and return to the CAD schematic editor window. Save the project file as “intermediate” (overwrite the old one).

The complete list of components and values as obtained form the single filter optimizations is shown above. It is observed that R12 is very small, 0.07295 ohms. This suggests that C10 and C11 could be combined into a single cap of 8.09465 uf and R12 eliminated. To do this without having to draw the entire schematic over again, first change the value of C10 to 8.09545. Then drag a resistor symbol over the Zobel symbol and release it. The Zobel should disappear and be replaced by the resistor. Next, click on this resistor and set its value to 100000 ohms. This effectively removes this branch from the circuit, as the 100K ohm resistor is much higher in impedance than any other shunt element. If you care to you can verify this by plotting the woofer response at node 7 before and after you make this change. I will not show this here. Once again, save the project file with these changes with the name “intermediate”.

The last thing to do in the crossover module is to create a project for the file that contains the SPL data with the woofer and tweeter measured at the same time. To do this, open a new project loading this driver file as the woofer. Create a simple circuit consisting of an input symbol connected directly to a driver symbol. Set the driver symbol type to woofer. Then open the frequency plot window and specify the driver type as a woofer and the “from” node as 0 and plot the result. You should see a plot, as below. Save this as a project file with a unique name.

### Sum Plot

The next step is to look at the system in the sum_plot module. So far we have optimized the HP and LP filters to yield the target acoustic response. However, this did not consider the position and offset of the drivers on the baffle. So, open the sum_plot module. Again, you do not need to close the box or crossover modules to do this. As I write this I have all three modules open, along with MS Word and MS Photo editor. The first thing we want to do in the sum_plot module is to check the driver offsets. To do this first open the project file with the you just created that contained the SPL data imported for the case where both the woofer and tweeter were measured at the same time with out a crossover. When open, you should see only a woofer present in sum _plot. Make sure the woofer and mic vertical position are the same. If they are, the “distance” shown to the right side of the mic vertical position slider should be 1.000. When you have this correct, open the frequency domain window and plot the system amplitude response. The plot should be identical to the one you plotted in the crossover module. Next, open the “intermediate” project file for the 2-way system. This should be the system project file you saved after you completed the tweeter filter optimization. Position the tweeter at a vertical (Y) position of 90 cm and the woofer at 77 cm. The X position should remain at 25 cm. Place the mic at 83 cm. Now, on the left side of the window use the Z slider under the woofer to position the woofer at 8mm. With these settings the “distance column should show 1.002 for the tweeter and 1.009 for the woofer. The path length difference is thus 7 mm as we determined when we imported the phase data. Now, click the plot button and choose “system amplitude”(the previous plot should still be there). When the plot box opens enter “0” for both the woofer and tweeter “from” nodes. Then click plot now. The result should appear as below with both plots overlaid.

If this is not the case, you didn’t import the SPL and phase data correctly and you will have to go back and start over with the phase importation. If you get this result click clear, plot, system amplitude, and when the plot box opens set the tweeter node to -4 and the woofer to 7 (assuming you set up the crossover with the same node numbers as I did) and plot the response. Then repeat the process with the tweeter node set to 4 instead of    –4.  You should see two plots as shown below. The plot with the notch should be the one corresponding to –4 and the flatter response the one with 4 for the tweeter node.  If this is correct, make sure both the woofer and tweeter nodes are positive, then save the project file again as “intermediate”.

For this particular system the results don’t look that bad. But we should still see if the system optimizer would improve the response. Go to the tool menu and select “system optimizer”. In the system optimizer, lock R6, the 4 ohm tweeter Zobel resistor, and R8 and L9, the 1.54 mH inductor and its series resistance, and R10, the 100K resistor. I locked R4 since I don’t want the shunt resistance in the tweeter Zobel to change, in particular to drop to a possible very low value. R10 was locked simply because it is a "non component” that won’t be in the actual crossover. R8 and L9 were locked to help “hold” the crossover point in the 3K range. Then click “use optimized” and you will see the system response plotted. Click “optimize” and in the optimization box set the optimization range to 200 to 20000. Set the attenuation to –3. Click the button “4. Optimize”. The code should now run the optimizer. When completed the response should look like that shown below.

Click “Use optimized” and make sure the optimized component values are loaded into the system (the “original values and the optimized values should be the same). Then return to the front panel from the “tools” menu. Select “frequency domain” and plot the system response, as before, with both 4 and –4 for the tweeter node. You should see a plot as shown below which clearly indicates an improvement in the depth of the notch and flatness of the response in the crossover region. Note that in this plot I have set the plot level to 87 dB instead of the 90 previously used. Make sure you save this project file. I suggest that you save it with a new name, “Final”.

We are not finished yet. Go to the tool menu and click 5 way polar plots. Check the woofer and tweeter boxes, set the driver separation to 13 cm and set the frequency to 3150. Then click “cross + driver”. You should get a plot like the one below, a classic L/R polar response pattern.

Now, go to the “tools” window and choose “Time domain”. Set the frequency to 100 and click plot. Choose “system plot”. Enter “1” in the sequence box and click plot now. The result should appear as below. This is a classic 4th order crossover response that also demonstrates the poor step response of this type of system. The tweeter rises and falls before the woofer even starts. This blurs transients, although it is a source of argument between those who claim they can hear the effect and those who claim it is inaudible.

Ok, again make sure you save this project file as a new project (I suggested “Final” as the name but its not final yet). We will now return to the crossover module for a little checking.

###### Returning to the Crossover Module

Open the crossover module and load the “Final” project file. All we will do here is set the component values to street values (values you can easily purchase). The figure below shows the component values from the system optimization done in sum_plot on the left along with design values on the right.

Note that C4 was determined to be 33.7 uf in the system optimization. However, in the table to the right I have replaced C4 with R4, a 0.0001 resistor. In other words, I have replaced C4 with a short circuit. Just as R11 will be omitted from the woofer circuit when built, C4 will also be omitted. You should drag and drop a resistor symbol over C4 to change it to R4, then assign a value of 0.0001 ohms to R4 and save the “final” project file one more time. The reason being that we are going to have to return to the sum_plot module again and check to be sure that this change doesn’t adversely affect the system response. (This is where it would be better if the component values could be changed manually in the sum_plot module. If they were, we wouldn’t of had to go back to the x-o module and then return again to sum_plot.)

###### A Final Check in Sum_plot

We must return to the sum_plot module and open the “final” project file once more. Upon doing this, open the frequency domain window and plot the system amplitude response with the tweeter node set at 4 and –4 once again. While you are there, also plot the impedance and impedance phase. You should have a plot that looks like this:

As you can see, there is still a deep null. However, it has moved up in frequency to 3400 Hz. For this exercise this is acceptable. If you so desire you can run the system optimizer and see if any significant improvement occurs, or if the crossover frequency shifts to a lower point. But this is about as far as I will take this design. You can see that the impedance remains above 6 ohms. At 10k Hz it is just under 8 ohms. The impedance phase reaches a minimum of –45 degrees at 3.6 kHz and is generally well behaved. If you like, impedance compensation can be added to reduce the peak at 2k Hz. I suggest a series RLC component connected between the input node (node 0) and ground. The values of R, L, and C should be 8 ohms, 0.22 mH and 28 uf, respectively. This should yield almost flat impedance between 300 and 10K Hz.

A final check of the polar response verifies the 3.4 k Hz crossover and retains the 4th order lobing pattern.

###### Measured Results

I had the parts so I figured why not build the final version. The results are below.

Here is the measured frequency response with normal and reversed tweeter phase. Note the agreement between this and the simulated result. The bumps in the response at 1.5k and 4.5k Hz and the null at 3.4k Hz are clearly evident. The scale here is 10 dB/division compared to 5dB/division in the SE plots.

Here is the measured impedance with (lower) and without (upper) the impedance compensation mentioned previously. As you can see, it takes a little time and effort, but it does work very well. The simulated results are for the most part identical to those that were finally measured.

###### Data files

The files that were included in this set of zipped files include the amplitude and impedance data that was used in these simulations. The files are LAUD/IMP type files and can be imported directly.

The SPL/phase files are as follows:

D25off.frd ------ Tweeter SPL and phase at 0.5 M, 15 degrees off axis horizontally

P13off.frd ------ Woofer SPL and phase at 0.5 M, 15 degrees off axis horizontally

Bothoff.frd ----- SPL and phase when both drivers are measured at the same time  
                                 with no crossover. Used to check path length difference for woofer

and tweeter.

Impedance files:

D25.zma ------- Tweeter impedance file

P13.zma ------- Woofer impedance file

###### Closing Remarks

While I have walked through a design here, it is not the only possible design that can be achieved with these drivers. I suggest that you try other crossover topologies and experiment with different optimization limits (frequency range, locked components, etc) to see the effect these choices have on the result. These are all tradeoffs and can lead to different results. There is not single “optimum” result, only “optimum” results corresponding to the choices and limitations you impose on the design process. In that sense, each design is completely unique. Only after a design is built, tested, and listened to extensively can one judge whether or not the design process was successful.

Another area which would be good to play around with in the importing of the SPL and phase data. For example, in this exercise I used 2nd order HP and 6th order LP slope correction on the woofer. The additional delay, relative to the tweeter, required to obtain a good phase match was found to correspond to a path length difference of 7 mm. But what if I had chosen a 2nd or 4th order LP slope correction? Could a phase match be found by altering the delay? And if so, would it change the result? I suggest that you return to the box module at some time after completing the design as presented here, open the woofer driver file and redo the transfer functions using a different LP slope correction. Make note of the new path length difference, and save the new woofer file. The go to sum_plot and open you final project file, but specify this new woofer file. When the project opens, adjust the woofer position to obtain the correct “distance” and check the frequency response result. If you do everything correctly I think you will gain some insight with regard to the roll of the slope correction and delay.

Hopefully, the design effort presented here will aid you in developing your own design philosophy, which should ultimately provide you with the experience and knowledge needed to execute successful designs with a minimum of effort. But remember there is always room for improvement.