Design Notes - CD4046 Phase-Locked Loop

By: Douglas Arduini

ARDUINI DESIGN & DEVELOPMENT

5803 Southpine Drive

San Jose, CA 95138

Phone/FAX 408/224-8120

E-Mail: doug@arduini.com

Dated last change: 6/24/95

Given, Simplified Notes:

1. R1, pin 11 = Sets maximum frequency when VCO IN, pin 9 is near VCC,

where: Fmax = 1 / [ R1 * (C1 + 32pf ) ]

2. R2, pin 12 = Sets minimum frequency when VCO IN, pin 9 is near VSS,

where: Fmin = 1 / [ R2 * (C1 + 32pf ) ]

With R2 open, minimum frequency is 0 Hz near VSS

3. R2 must be > R1 for the VCO output frequency to change with dv at VCO IN, pin 9.

4. FL = Frequency lock range is between Fmax & Fmin, but should set R1 & R2 to bracket FL  near middle of VCC

and VSS for tolerance variations and minimize VCO dynamic voltage slew range.

The VCO slew range for maintaining lock is effected by the phase discriminator low-pass filter slew rate

and gain of the VCO internal amplifier (and buffer amp if used ).

4. FC = Frequency capture range to achieve frequency lock FL is dependent on the phase comparator output used.

A. Using phase comparator output I, pin 2 into a low pass filter, FC = FL

B. Using phase comparator output II, pin 13 into a low pass filter, FC is much less than FL, dependent on

the low pass filter time constant,

where: 	FC»1p2pFLR3C2

Notes: 	1. R1, R2, R3, C1, C2 are references to National Semiconductor data sheet schematic figures.

2. Above reference of  FC may be referenced as 2 FC in the National Semiconductor data sheet schematic

figures.

Experienced, Simplified Notes:

1. Phase comparator (discriminator) I versus comparator (discriminator) II use:

A. Faster frequency capture

B. Narrow lock range from Fmin and Fmax limits set by R2,3, C3.

C. More stable phase stability  (less phase jitter when locked)

a. Comparator output voltage is PWM for averaging filtering, more stable without need for R4 damping resistor, at least at very low frequencies (10Hz) versus high frequencies (90kHz).