The following are the equations I used in Tesla Coil Helper.

I include them here for your convienience. At the end of this file

is some other good info.

=======================================================================

EXPLAINATION OF EQUATIONS:

Here are some real handy equations. They are very simple and easy to

use.

x = Multiply by

/ = divide by ( may also use line seperating terms above and below

line as in standard mathematics).

( ) = Terms in parentheses should be calculated first as in standard

algebraic equations.

pi = 3.141592654. The circumferance of a circle divided by it's

diameter will always give you this constant.

2

Z   = means the term "Z" multiplied by itself one time,"Z x Z".

Note: In some cases I do not use the symbol "x" but instead simply

put the terms close together, example: "LC" instead of "L x C".

This is standard for algebraic equations and means "multiply by".

=======================================================================

MATH FOR TESLA COILS

1. Determine your neon sign transformer (or other transformer's)

Impedence:

E

Z  =  ---

I

Z = Impedence

E = volts

I = current in Amps

Note: divide milliamps by 1000 to get Amps. 30 milliamps = .030 Amps.

The Impedence of the primary capacitor should match the Impedence of the

transformer at 60 Hz (60 Hz is the AC cycle rate of common household

wall sockets, at least in America).

2. To match Impedence and determine capacitor value:

1

C =   -------------------

2 x pi x Z x .00006

C = capacitance in microfarads needed for primary capacitor.

Z = Impedence from equation one (Transformer Impedence)

pi = 3.141592654

Note: The .00006 is the 60 Hz AC, if you live outside the US then

substitute your cycle rate.

Next you need to find the Reactance of the primary capacitor at the

frequency you have choosen. Many times the frequency is decided by the

length of wire used on the secondary coil. See below for equations that

determine frequency by length of wire used on secondary.

When we find the Reactance , we can then find your needed Inductance

for the Primary coil.

3. To determine Reactance of capacitor:

1

X(C) = ---------------

2 x pi x C x F

X(C) = capacitor Reactance

C = Capacitor value in microfarads, from equation 2)

F = Frequency in Mhz (megahertz)

pi = 3.141592654

Note: To convert kilohertz to megahertz simply divide by 1000.

190 Khz = .190 Mhz

4. To determine the Inductance needed for Primary coil:

Set X(L) = X(C)

X(L)

L = ------------

2 x pi x F

L = Inductance in microhenries needed for Primary Coil.

To get millihenries divide the answer by 1000.

X(L) = Reactance from equation 3, same as X(C).

F = frequency in Megahertz.  Divide Khz by 1000 to get Mhz.

pi = 3.141592654

Now you know the values for your capacitor and primary coil. These

values will give you the best ringing for your circuit (ie. more bang

for your buck)! Use the equations below to finnish the project.

Note: Many people don't go to the trouble to work these equations out.

They simply slap the parts together and then try to tune. If you work

the equations out first you will save lots of time in tuning, you will

at least be in the right ball park! Also, just because you worked it

out on paper that doesn't mean it will work the first time you plug it

in. Trial and error is a large part of the Tesla Coil hobby!

=======================================================================

CAPACITORS IN PARALLEL:

|      |

|__C1__|

|      |

|__C2__|

|      |

|__C3__|

|      |

Capacitance = C1 + C2 + C3, etc...

Maximum voltage rating will be equal to the voltage rating of the

lowest voltage capacitor of the group.

=======================================================================

CAPACITORS IN SERIES:

|              |

|__C1__C2__C3__|

|              |

|              |

1     1     1

Capacitance = 1 / --- + --- + ---, etc...

C1    C2    C3

The total capacitance of several capacitors in series will always be

LESS than that of the smallest capacitor.

Total voltage rating increases with number of capacitors in series.

Simply add the voltage ratings together.

When capacitors are placed in series to increase voltage rating they

should have the same capacitance and voltage rating else voltages will

divide unevenly, most likely causing failure.

=======================================================================

EQUATION 1:  PLATE TYPE CAPACITORS

Capacitance (in picofarads) = (0.224 KA / d) (N-1)

0.224 x Dielectric Strength x Area of plate

C = -----------------------------------------  x (Number of plates - 1)

distance between plates in inches

Note: to convert picofarads to microfarads divide by one million

( 1000000 ).

=======================================================================

EQUATION 2: LEYDEN JAR or SALT WATER TYPE CAPACITORS (jar/bottle type)

2

C = .0884 k ( pi  r   + 2  pi  r  l )

------------------------------

1,000,000 t

C = Capacitance in microfarads

k = dielectric strength

r = jar radius in centimeters

l = height of the jar portion used (in centimeters)

t = thickness of the jar wall in centimeters

pi = 3.141592654

2

r   = r x r  (radius squared)

=======================================================================

EQUATION 3: FREQUENCY OF A CIRCUIT

1

f =  --------------------

__________

2 pi   / L C

f = frequency in cycles per second

L = circuit inductance in henries

C = circuit capacitance in farads

pi = 3.141592654

_________

The symbol " /          " means the square root

For a result "f" in Khz: enter "C" in microfarads, "L" in microhenries

and multiply result by 1000.

=======================================================================

EQUATION 4: INDUCTANCE OF A FLAT PANCAKE COIL

Picture a 1 inch flat ribbon that is about 30 feet long. Now, roll

that ribbon into a spiral that has all its sides about 1/2 inch apart.

Most common material is Aluminum Roof Flashing. Use plastic bolts to

hold sections of strips together if you have short pieces of ribbon.

This makes a good mechanical connection (you can't solder aluminum).

center axis

|

| | | | |    |    | | | | |       <---cross section

|                        of flat spiral

|---A----|    |---W---|           coil.

|

2         2

a     x   n

L = ---------------

8 a   +   11w

L = inductance in microhenries.

a = average radius in inches as measured from the central axis to

the middle of the winding.

n = number of turns in the winding.

w = width of the coil in inches.

Note: Make sure you measure "a" from center axis - the very middle

of your secondary sitting inside of your primary.

=======================================================================

EQUATION 5: NUMBER OF TURNS FOR A HELICAL PRIMARY

_________________________

/

N =     /  L  [( 9 x R) + (10 x H)]

/  --------------------------

/            2

\/            R

N = Number of turns needed.

L = inductance in microhenries desired.

R = radius (inches).

H = height (inches).

_________

The symbol " /          " means the square root, in this case of whole

equation.

=======================================================================

EQUATION 5: LENGTH OF WIRE NEEDED FOR DESIRED FREQUENCY OF COIL

300,000

L =     -------  / 4 x (3 / .9144)

f

f = frequency, in Khz, that is desired for coil.

L = length of wire needed, in feet, for desired frequency.

/ = divided by.

Note - 300,000 is the speed of light in Kilometers per second. the

term "3/.9144" is a conversion factor to turn meters to feet.

You don't have to understand this. Just thought I would tell

those who were wondering.

=======================================================================

EQUATION 6: FRQUENCY OF COIL

300,000

f =  ------------------------------

T x W x pi x (.9144 / 36) x 4

f = frequency of coil in Khz

T = number of turns on coil

W = width of the coil in inches

pi = 3.141592654

=======================================================================

EQUATION 7: CAPACITANCE OF A SPHERE IN SPACE

R

C =   -------

9

9 x 10

C = capacitance in Farads

R = radius in meters

9

9x10  = 9,000,000,000

6

Note: To convert Farads to microfarads simply multiply by 10   or in

other words by 1,000,000.

=======================================================================

EQUATION 8: CAPACITANCE OF A SPHERE SUSPENDED IN A DIELECTRIC

K x R

C = ---------

9

9 x 10

C = capacitance in Farads

R = radius in meters

K = dielectric constant

6

Note: To convert Farads to microfarads simply multiply by 10   or in

other words by 1,000,000.

=======================================================================

EQUATION 9: CAPACITANCE OF A TOROID

___________________

/    2

C =(1+ (0.2781 - d2/d1)) x  2.8  x      /  2 pi  (d1-d2)(d2/2)

/   -------------------

\/      4 pi

C = capacitance in picofarads (+- 5% )

d1 = outside diameter of toroid in inches

d2 = diameter of cross section (cord) of toroid in inches

Equation courtesy of Bert Pool

=======================================================================

TESLA COIL SCHEMATIC                                  --------

|      | TC

--------

O

O

SG   SC      FCC     PC   SG        P-COIL     O S-COIL

O---------------OOOOO---------> <-----    O        O

NST  O    |    |              |           |    O        O

O    |    |              |           |    O        O

-----O||O    *   ---            ---          |--->O        O

O||O                                         O        O

-----O||O    *   ---            ---               O        O

110   | O    |    |              |                O        O

Volts | O    |    |              |                O        O

| O---------------OOOOO---------------------|        |

|                                                    |

|                                                    |

Gnd                                                  Gnd

NST = Neon Sign Transformer, 110 volts primary, 15,000 volt secondary

at 30-60 miliamps.

SG = Safty Gap. A spark gap to insure your transformer doesn't get fried.

SC = Safty Capacitor. 300-500 picofarad rated at 50 KV.

FCC = Ferrite Core Choke. This prevents real high voltages from coming

back towards your transformer. It also seperates the capacitors.

PC  = Primary Capacitor. Normally .01-.02 microfarads rated 50 KV AC.

SG  = Spark Gap.

P-COIL = Primary Coil.

S-Coil = Secondary Coil.

TC  =  Terminal Capacitor. The big ball or coffee can on top.

Gnd = Ground.

Note: This is one of several possible schematics. It's just the one I

happen to use.

=======================================================================

Bibliography:

Books:

-------

Theory and Applications of Electricity and Magnetism, Charles A.

Culver, 1947, McGraw Hill Book Company, Inc.

Concept In Physics, Third edition, Frankln Miller Jr., Thomas J.

Dillon, Malcom K. Smith, 1980, Harcourt Brace Jovanovich, Inc.

* Tesla Coil Secrets, R.A. Ford, 1985, Lindsay Publications Inc.

* Tesla Coil, George Trinkaus, 1989, High voltage Press @Lindsay

Publications.

* High frequency Apparatus, Thomas Stanley Curtis, 1916, Lindsay

Publications.

Allied Electronics Data Handbook, 1970

* Tesla Coil Design Manual, J.H. Couture, 1992

Radio Amateur's Handbook, 1972

Elements of Physics, 1964

Articles:

----------

Popular Electronics, Make Your Own High Voltage Capacitors,

Anthony charlton.

* These books I recomend for anyone serious about building Tesla Coils.

See:      Lindsays Publications

P.O. Box 538

Bradley, Il 600915

phone (815) 935 - 5353

fax   (815) 935 - 5477

Ask for their Electrical or Technical catalogs which are free. You

will not be disapointed!

=======================================================================

WARNING:

Only people who are experienced with High Voltage devices should

attempt to build Tesla Coils. They can be very deadly if you don't

know what your doing. Remember, if they find you on the floor

turning blue and frothing at the mouth - THERE IS NO SECOND CHANCE!

A FEW Safety Tips:

1. Don't ever touch the machine when it is pluged in.

2. Use a safe methode to short out the primary capacitor after the

machine has been run.

3. Don't ever get close to a running Tesla coil, the Primary can

shoot hot white arcs at you that will kill you instantly!

4. Always have a small fire extinguisher close by.

5. Always use kickback preventer circuits so you don't send

15,000 volts back through the wall!

6. Pets, children, and irresponsible adults should be kept away from

your machine intirely!!!

7. Read many books on Tesla Coils and other High Voltage devices

and learn as much as you can about High Voltage Safety!

=======================================================================

Special thanks to Bert Pool who caught some glaring errors in the second

draft and who contributed with an equation of his own.

I wrote this file because I felt there was a need for some real

information for those of us who actually build Tesla Coils, as opposed

to those who just talk of building them (Grin). I will be adding to this

file from time to time, so watch for updates. I hope it helps you out!

Jerry Gore, Member of Tesla Coil Builders Association (TCBA),

International Tesla Coil Society, and the North Dallas

Texas Tesla Coil Association.

P.S. Let us not forget the words of Tesla," Let the future tell the

truth and evaluate each one according to his work and accomplishments.

The present is theirs, the future, for which I really worked, is mine".

======================= END OF FILE ===================================