FRONTEND 4062 FY5		3X 8501

3X 9957

TARGET - SPECIFICATION ELECTRICAL DATA

1. Description:

The frontend 4062 FY5 is specially designed for multimedia applications. Reception standard is Pal I . The frontend includes a hyperband tuner which covers the frequency range from 45 to 860 MHz and an IF-part with SAW-filter, IF-amplifier, video and sound demodulator. So the CVBS signal is available at the video output terminal, the audio signal at the audio output terminal. Also a 2nd IF output is provided, which allows external sound demodulation for NICAM reception.

The reception frequency range is divided in VHF low, VHF high and UHF. Bandselection and tuning is done via I²C-bus, completely.

Also a digital AFC-function can be realized, because the AFC-voltage, generated by the IF-demodulator is fed to an analog/digital converter which is included in the PLL-IC and readable via I²C-bus.

2. Mechanical Characteristics:

2.1. Dimensions: 	according drawing 3X 8501GZ

2.2. Weight:	ca. 51 g

2.3. Types

| Tunertyp | 3X 8501 | 3X 9957 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sockettyp | MINI PHONO | IEC |  |  |  |  |
| Socketlength | 8.5 mm | 14.0 mm |  |  |  |  |
| Height | 29.1 mm | 29.1 mm |  |  |  |  |

3. Working Data:

3.1. Reception Standard:				Pal I

3. 2. Frequency Range:

VHF low			ch  A   ...	 S 6			  45.75 MHz  ...	140.25 MHz

VHF high			ch S 7 ...	 S 41		147.25 MHz  ...	463.25 MHz

UHF			ch 21  ...	 E 69		471.25 MHz  ...	855.25 MHz

Margin:

VHF low		ch A    ...	S 6 		-0.5 MHz / +1 MHz

VHF high		ch S 7 ...	S 41		-7 MHz / +2 MHz

UHF		ch 21   ...	E 69		-8 MHz / +3 MHz

Recommended take over frequencies:

VHF low / VHF high			144 MHz

VHF high / UHF 			467 MHz

Frequency referred to channel center.

IF:

picture carrier:						38.90 MHz

sound carrier :						32.90 MHz

NICAM sound carrier:					32.35 MHz

Oscillator operates above received frequency.

3.3. Supply voltage:

Supply voltage VS1			5 V +/- 5%	max. 200 mA

Supply voltage VS2		33V +/- 5%	max. 1.7 mA

3.4. Input impedance:

VHF/UHF common				75 W, unbalanced

3.5. Temperature:

(measured in slowly moved air)

Operating temperature:				   0  ... 60 °C

Storage temperature:				-25  ... 60 °C

4. Test conditions:

If not otherwise noticed all data are hold under following conditions:

Measurement tolerance:	10 % or 1 dB

Ambient temperature:	25 °C  +/- 3°

Supply voltages:	V S1, V S2 +/- 5%

5. Tuner Data:

5.1. Voltage Gain:

Voltage gain is measured between antenna input and IF1-Mp and IF2-Mp.

For this measurement the input is loaded with 75, the output is loaded

with a test circuit according diagram.

min.	typ.	max.	unit

ch A ... ch 69		40		52	dB

5.2. Noise Figure:

VHF low						 8.0	dB

VHF high						 8.0	dB

UHF						10.0	dB

5.3. VSWR:

VHF low						4.0

VHF high						4.0

UHF						4.0

Referred to channel center frequency.

5.4. AGC-Range:

min.	typ.	max.	unit

VHF low				50			dB

VHF high				45 			dB

UHF				40 			dB

5.5. IF-Rejection:

VHF low						50 		dB

VHF high						60 		dB

UHF						60 		dB

5.6. Image-Rejection:

ch A   ... 	ch S 6			60 		dB

ch S 7 ... 	ch S 20			60 		dB

ch S 21...	ch S 41			60 		dB

ch 21 ...	ch E 69			53 		dB

5.7. RF Tilt:

The amplitude difference between top of frequency response curve and any

frequency between picture and sound carrier will not exceed 4 dB at nominal gain.

5.8. Test circuit for voltage gain:

test circuit attenuation: 31.2 dB

5.9. Test circuit for noise figure:

6. Output parameter:

6.1. Video output:

Output signal type: CVBS

Conditions:		Testsignal	min.	typ.	max.  	unit

Ant. input level 66 dBmV

20% residual carrier

CVBS - Output level:				1.5	1.8 	2.1	V (p-p)

Sync bottom dc level:					1.35		V

Load impedance					1			k

Load capacitance							20	pF

Video S/N (unweighted):

Black burst

VHF						43			dB

UHF	(70dBmV input level)			43			dB

Frequency response:		(sin x)/x

Ref.: 0.2 MHz

1 MHz				-1.0		 1.5	dB

2 MHz				-1.5		 2.0	dB

3 MHz				-2.5		 2.0	dB

4 MHz				-3.5		 1.0	dB

4.43 MHz			-7.0		-1.0	dB

5 MHz					-12		dB

Differential gain				Modulated 5 step staircase			5	% (p-p)

Differential phase			Modulated 5 step staircase			5	 (p-p)

6.2. Sound output:

Conditions:		Testsignal	min.	typ.	max.  	unit

Ant.input level: 66 dBmV

Video signal: color bar

PC / SC : 10dB			AF1: 1kHz, 27 kHz deviation,

50s preemphasis

Output level AF1:		AC			1.2		V (p-p)

DC			2.0		V

Load impedance:				1.5			k

Measurements with 50s deemphasis:

AF1 - level	:					250	300	350	V rms

THD :									0.4	%

S/N       (CCIR weighted) :			44			dB

SINAD  ( 22Hz ... 22kHz )  :				50		dB

Frequency response:

(6 kHz deviation)

40 Hz ... 15 kHz			-1		1	dB

6.3.  2nd IF output

AC level of  6.0   MHz:				100		mV (p-p)

6.55 MHz: (PC / NICAM-Carrier : 20dB)		  50		mV (p-p)

Load impedance			0.5			k

Note: Short circuit at pin 18 or pin 21 can damage internal circuits.

7. I²C bus

7.1 Write data format

|  | MSB |  |  |  |  |  |  | LSB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Address byte | 1 | 1 | 0 | 0 | 0 | MA1 | MA0 | R/W | A |
| Divider byte 1 | 0 | n14 | n13 | n12 | n11 | n10 | n9 | n8 | A |
| Divider byte 2 | n7 | n6 | n5 | n4 | n3 | n2 | n1 | n0 | A |
| Control byte 1 | 1 | CP | T2 | T1 | T0 | RSA | RSB | OS | A |
| Control byte 2 | P7 | P6 | P5 | P4 | P3 | P2 | P1 | P0 | A |

A	:	Acknowledge

7.1.1 Address selection

R/W	:	Read / Write mode	0 : Write mode

| MA1 | MA0 | Address | Voltage at Pin 11 |
| --- | --- | --- | --- |
| 0 | 0 | C0 | ( 0 to 0.1 ) * VS1 |
| 0 | 1 | C2 | always valid |
| 1 | 0 | C4 | ( 0.4 to 0.6 ) * VS1 |
| 1 | 1 | C6 | ( 0.9 to 1 ) * VS1 |

7.1.2 Oscillator frequency and divider byte calculation:

n14, ... n0		Divider bits  (Scaling Factor)

f osc	=   fw  + 38.9 MHz		( f osc : Local oscillator frequency;   fw  : wanted frequency )

SF	=   f osc / (f ref * 8)   		( f ref  : Reference divider )

| RSA | RSB | Reference divider | Min. tuning step [kHz] | fref [kHz] |
| --- | --- | --- | --- | --- |
| 1 | 1 | 512 | 62.5 | 7.8125 |
| X | 0 | 640 | 50.0 | 6.25 |
| 0 | 1 | 1024 | 31.25 | 3.90625 |

SF = 16348 * n14  +  8192 * n13  +  4096 * n12  +  2048 * n11  +  1024 * n10  +  512 * n9  +  256 * n8  +

128 * n7  +  64 * n6  +  32 * n5  +  16 * n4  +  8 * n3  +  4 * n2  +  2 * n1  +  n0

7.1.3. Control byte settings

Control byte 1 :

CP	:		Charge pump		1 :	 fastest tuning

T2	:		Test mode bit		0 :	 normal mode

T1	:		Test mode bit		0 : 	 normal mode

T0	:		Test mode bit		1 :	 normal mode

RSA	:		Reference divider		 according 7.1.2

RSB	:		Reference divider		 according 7.1.2

OS	:		PLL disabling		0 :	 normal mode

Control byte 2  (Bandselection) :

| Band | P7 | P6 | P5 | P4 | P3 | P2 | P1 | P0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UHF | X | 0 | X | X | 0 | 0 | 0 | 1 |
| VHF high | X | 0 | X | X | 0 | 1 | 0 | 0 |
| VHF low | X | 0 | X | X | 0 | 0 | 1 | 0 |

7.2. Read data format

|  | MSB |  |  |  |  |  |  | LSB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Address byte | 1 | 1 | 0 | 0 | 0 | MA1 | MA0 | R/W | A |
| Status byte | POR | FL | I2 | I1 | I0 | A2 | A1 | A0 | A |

R/W	:	Read / Write mode			1 : Read mode

POR 		: 		Power on reset flag 					1 : at power on

FL 		:  	In lock flag 						1 : PLL is locked

I2,I1, I0	:		Digital levels for I/O ports P7, P5 and P4

A2, A1, A0	:		Digital output of 5-level ADC for AFC function

Value for correct tuning:				A2 = 0,  A1= 1,  A0 = 0

|  |  | NAME |  | Zahm |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DATE |  | 06.12.95 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | REV.: |  | 01 | 02 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | FÄM.- NO. |  | 559 | 54/96 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | DATE |  | 7.12.95 | 19.04.96 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NAME |  | Zahm | Zahm |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | SIGNATURE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |