# COAXIAL CABLE SPECIFICATIONS  Barend Hendriksen       1

| RG/U | Man. | NEMAL NO. | CENTER CONDUCTOR | DIELECTR | Veloc. % | SHIELDING | JACKET (0.0. in.) | WEIGHT Un./M | NOMINAL IMPEDANCE lOhimI | NOMINAL CAMCIIANU (PF/ni | MAXIMUM VOLTAGE NgMt/IIMS) | OPERATING TEMPERATURE °C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | Nemal | 1000 | 18 BC | FP | 78 | BC 95% | PVC .270 | 32 | 75.0 | 17.3 | 400 | -40 to +80 |
| 6 | Nemal | 1001 | 18CCS | FP | 78 | At. Foil +40%AI. | PVC .275 | 29 | 75.0 | 16.5 | 400 | -40 to +80 |
| 6 | Siamese | 1004 | 18CCS | FP | 78 | Al. Foil + 40% Al. | PVC .275x.595 | 59 | 75.0 | 16.5 | 400 | -40 to +80 |
| 6 | Nemal | 1007 | 18.CCS | FP | 78 | Al. Foil + 60% A I. | PVC .275 | 29 | 75.0 | 16.5 | 400 | -40 to +80 |
| 6 | Messenger | 1008 | 18CCS | FP | 78 | Al. Foil + 40% Al. ¦ | PVC .275 x .430 | 41 | 75.0 | 16.5 | 400 | -40 to + 80 |
| 6 | Flooded | 1010 | 18CCS | FP | 78 | Al. Foil +40%AI. | PVC .275 | 28 | 75.0 | 16.5 | 400 | -40 to+80 |
| 6A | Nemal | 1005 | 21 CCS | PE | 66 | (2) BC 95% | PVC .336 | 70 | 75.0 | 20.5 | 2700 | -40 to +80 |
| 8 | Jan C-17 | 1100 | 13 BC (7x21) | PE | 66 | BC 95% | PVC .405 | 106 | 52.0 | 29.5 | 4000 | -40 to +80 |
| 8 | Nemal | 1101 | 13 BC (7x21) | FP | 78 | BC 80% | PVC .405 | 102 | 50.0 | 26.0 | 600 | -40 to +80 |
| 8 | Nemal | 1102 | 11 BC (19x24) | FP | 78 | BC 95% | PVC .405 | 107 | 50.0 | 26.0 | 600 | -40 to + 80 |
| 8X | Nemal | 1110 | 16 BC (19x28) | FP | 78 | BC 95% | PVC .242 | 35 | 50.0 | 28.0 | 300 | -40 to+80 |
| 11 | Jan C. 17 | 1200 | 18 TC (7 x 26) | PE | 66 | BC 95% | PVC .405 | 96 | 75.0 | 20.5 | 4000 | -40 to + 80 |
| 11 | Nemal | 1201 | 14 BC | FP | 78 | BC 95% | PVC .405 | 92 | 75.0 | 17.3 | 600 | -40 to +80 |
| 11 | Messenger | 1202 | 14 CCS | FP | 78 | Al. Foil + 40% Al. | PVC .405 x .631 | 89 | 75.0 | 16.5 | 600 | -40 to +80 |
| 11 | Flooded | 1210 | 14 CCS | FP | 78 | Al. Foil + 40% Al. | PVC .405 | 62 | 75.0 | 16.5 | 600 | -40 to + 80 |
| 11A | Mil C-17 | 1205 | 18 TC (7 x 26) | PE | 66 | BC 95% | NCV .405 I | 96 | 75.0 | 20.6 | 5000 | -40 to +80 |
| 14A | - | See R | G217 | - | - | - | - | - | - | - | " | - |
| 17 | - | See R | G218 | - |  | - | - | - | - | - | - | - |
| 22 B | Nemal | 1920 | (2)TB (7 x.0152) | PE | 66 | TC 95% | NCV .420 | 151 | 95.0 | 16.0 | 1000 | -40 to +80 |
| 55A | - | See R | G223 | - | - | - | - | - | - | - | - | - |
| 55B | Jan C-17 | 1401 | 20 SC | PE | 66 | (2) TC 95% | PE .206 | 33 | 53.5 | 28.5 | 1900 | -55 to +80 |
| 58 | Jan C-17 | 1410 | 20 BC | PE | 66 | TC 95% | PVC. 195 | 29 | 53.5 | 28.5 | 1900 | -40 to+80 |
| 58 | Nemal | 1412 | 20 BC | FP | 78 | BC95% | PVC. 195 | 29 | 50.0 | 26.0 | 2000 | -40 to +80 |
| 58A | Jan C-17 | 1420 | 20 TC (19x32) | PE | 66 | TC 95% | PVC. 195 | 29 | 52.0 | 28.5 | 1900 | -40 to +80 |
| 58A | Nemal | 1421 | 20 TC (19x32) | FP | 78 | TC 95% | PVC. 195 | 23 | 50.0 | 26.0 | 200 | -40 to +80 |
| 58C | Jan C-17 | 1425 | 20 TC (19x.0071 | PE ) | 66 | TC 95% | NCV.195 | 29 | 50.0 | 30.8 | 1900 | -40 to +80 |
| 59 | Jan C-17 | 1500 | 22 CCS | PE | 66 | BC 95% | PVC .242 | 32 | 73.0 | 21.0 | 2300 | -40 to +80 |
| 59 | Nemal | 1502 | 22 CCS | FP | 78 | BC 70% | PVC .242 | 30 | 79.0 | 16.3 | 300 | -40 to +80 |
| 59 | Nemal | 1504 | 20 CCS | FP | 78 | Al. Foil +40%AI. | PVC .238 | 23 | 75.0 | 16.5 | 300 | -40 to +80 |

COAXIAL CABLE SPECIFICATIONS Barend Hendriksen          2

| RC/U type | STEC. | NEMAL FMT NO. | CENTRE CONDUCTOR | DIELECTRIC | VELOCITY % | SHIELDING | JACKET o/d inch | WEIGHT lbs per 1000ft = 304,8 m | NOMIN. IMPED. (Ohm.) | NOMINAL CAP. Pf/Ft | MAX VOLT | OPER. TEMP. "C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59 | Flooded | 1506 | 20CCS | FP | 78 | Al. Foil + 40% Al. | PVC .238 | 22 | 75.0 | 16.5 | 300 | -40 to+80 |
| 59 | Nemal | 1508 | 20CCS | FP | 78 | Al. Foil + 60% Al. | PVC .238 | 23 | 75.0 | 16.5 | 300 | -40 to+80 |
| 59 | Nemal | 1510 | 22 BC (7 x 30) | FP | 78 | BC95% | PVC .242 | 27 | 75.0 | 17.3 | 300 | -40 to +80 |
| 59 | Nemal | 1512 | 20CCS | FP | 78 | BC95% | PVC .238 | 26 | 75.0 | 16.5 | 300 | -40 to +80 |
| 59 | Siamese | 1530 | 20CCS | FP | 78 | Al. Foil + 40% Al.. | PVC .238 x .521 | 46 | 75.0 | 16.5 | 300 | -40 to +80 |
| 59 | Messenger' | 1540 | 20CCS | FP | 78 | Al. Foil + 40% Al. | PVC .238x .393 | 35 | 75.0 | 16.5 | 300 | -40 to+80 |
| 59 | Siamese Flooded | 1550 | 20CCS | FP | 78 | Al. Foil +40%AI. | PVC .238 x .521 | 45 | 75.0 | 16.5 | 300 | -40 to +80 |
| 59 | Wang | 1560 | .023 CCS | PE | 66 | BC95% | PVC .238 x .478 | 75 | 75.0 | 20.5 | 2300 | -40 to +80 |
| 59B | Mil C-17D | 1520 | .023 CCS | PE | 66 | BC95% | NCV .242 | 32 | 75.0 | 20.6 | 2300 | -40 to+80 |
| 62A | MJIC-17D | 1600 | 22 CCS | ASP | 84 | BC95% | NCV .242 | 38 | 93.0 | 13.5 | 750 | -40 to +80 |
| 71 | Mil C-17D | 1620 | 22CCS | ASP | 84 | (2) TC 95% | PVC. 250 | 46 | 93.0 | 13.5 | 750 | -40 to + 80 |
| 108A | Mil C-17D | 1910 | (2) .0378 (7X .0126) TC | PE | 66 | TC95% | NCV .235 | 32 | 78.0 | 19.6 | 1000 | -40 +to80 |
| 115A | Mil C-17D | 1700 | .0840 (7x.28) SC | PTFE TAPE | 69 | (2) SC 95% | FG .415 | 180 | 50.0 | 29.4 | 5000 | -55 to + 250 |
| 122 | Mil C-17D | 1460 | .030 (7x.005)T | PE C | 66 | TC 95% | NCV. 160 | 16 | 50.0 | 29.4 | 1900 | -40 to +80 |
| 141A | Mil C-17D | 1715 | .039 SCCS | PTFE | 69 | SC 95% | FG .190 | 36 | 50.0 | 29.4 | 1900 | -55 to + 250 |
| 142B | Mil C-17D | 1705 | .039 SCCS | PTFE | 69 | (2) SC 95% | FEP.195 | 50 | 50.0 | 29.4 | 1900 | - 55 to + 200 |
| 174 | MilC-17D | 1450 | .0189 CCS (7 x .0063) | PE | 66 | TC 95% | PVC. 100 | 8 | 50.0 | 30.8 | 'we | -40 to+80 |
| 178B | Mil C-17D | 1710 | 30 SCCS (7 x 38) | PTFE | 69 | SC 95% | FEP .072 | 5 | 50.0 | 29.4 | TJOO | -40 to+80 |
| 179B | MilC-17D | 1712 | 30 SCCS (7 x 38) | PTFE | 69 | SC 95% | FEP.100 | 10 | 75.0 | 19.5 | •I 200 | -55 to + 200 |
| 180B | MilC-17D | 1714 | 30 SCCS (7 x 38) | PTFE | 69 | SC 95% | FEP.145 | 19 | 95.0 | 15.4 | 1500 | -55 to + 200 |
| 188A | MilC-17D | 1720 | 26 SCCS (7 x.0067) | PTFE | 69 | SC 95% | PTFE .110 2. &»v^»-> | 11 | 50.0 | 29.4 | I-200 | -55 to + 250 |
| 196A | MilC-17D | 1725 | 30 SCCS (7 x 38) | PTFE | 69 | SC 95% | PTFE .080 | 6 | 50.0 | 29.4 | •:000 | -55 to + 250 |
| 213 | MilC-17D | 1130 | BC (7 x .0296) | PE | 66 | BC95% | NCV .405 | 99 | 50.0 | 30.8 | 5000 | -40 to+80 |
| 214 | MilC-17D | 1140 | SC (7 x .0296) | PE | 66 | (2) SC 95% | NCV .425 | 126 | 50.0 | 30.8 | 5000 | -40 to +80 |
| 214 | Nemal | 1150 | TC (7 x .0296) | PE | 66 | (2) TC 95% | NCV .425 | 126 | 50.0 | 30.8 | 5000 | -40 to +80 |
| 217 | MilC-17D | 1310 | .106BC | PE | 66 | (2) BC 95% | NCV.545 | 201 | 50.0 | 30.8 | 7000 | -40 to +80 |
| 218 | MilC-17D | 1800 | .195 BC | PE | 66 | BC95% | NCV .870 | 460 | 50.0 | 30.8 | 1 1000 | -40 to+80 |

Coaxial Cable Specifications                     3

| RG/U type | SPEC. , | NEMAL PART NO. | CENTER CONDUCTOR | DIELECTRIC | VELOCITY % | SHIELDING | JACKET (O.D.In.) | WEIGHT Lb/1000Ft | NOMINAL IMPEDANCE (Ohrns) | NOMINAL CAP. PF/fT | MAXIMUM VOLTAGE (Volt/RMS) | OPERATING TEMP (°C) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 223 | MilC-170 | 1470 | .035 SC | PE | 66 | (2) SC 95% | NCV .216 | 34 | 50.0 | 30.8 | 1900 | -40 to+80 |
| 225 | MIIC-17D | 1760 | .0936 SC (7 x.0312) | PTFE | 69 | (2) SC 95% | FG .430 | 180 | 50.0 | 29.4 | 5000 | -55 to + 250 |
| 316 | MilC-17D | 1730 | 26 SCCS (7 x.0067) | PTFE | 69 | SC95% | FEP.102 2.^4 ^ | 12 | 50.0 | 29.4 | 1200 | -55 to + 200 |
| 393 | MilC-170 | 1770 | .0936 SC (7 x.0312) | PTFE | 69 | (2) SC 95% | FEP.390 | 165 | 50.0 | 29.4 | 5000 | -55 to + 200 |
| Twinax | Nemal | 1900 | (2) 20 (7 x 28) B/T | PE | 66 | TC95% | PVC .330 | 72 | 100.0 | 15.5 | 1000 | -40 to+80 |
| Belden 9913 | Belden | 1180 | .108BC | ASP | 89 | Al. Foil + 88% TC | PVC .405 | 97 | 50.0 | 24.0 | 600 | -40 to +80 |
| Belden 8281 | Belden | 1185 | 20 BC | PE | 66 | (2) TC 96% | PE .304 | 73 | 75.0 | 21.0 | 2900 | -55 to +80 |
| Belden 8218 | Belden | 1190 | 27CCS (7 x 35) | PE | 66 | TC 93% | PVC. 150 | 13 | 75.0 | 20.5 | 2000 | -40 to +80 |
| Triax 11 | Nemal | 1820 | BC (7 x.0159) | PE | 66 | (2) BC 95% | PVC .500 | 150 | 75.0 | 20.5 | 5000 | -55 to +80 |
| Triax 58 | Nemal | 1830 | .038 BC (7 x.0126) | PE | 66 | (2) BC 95% | PVC .245 | 41 | 50.0 | 30.8 | 2000 | -55 to +80 |
| Triax 59 | Nemal | 1840 | .023 BC | PE | 66 | (2) BC 95% | PVC .325 | 66 | 75.0 | 20.5 | 2300 | -55 to +80 |

MATERIALS USED IN COAXIAL CONSTRUCTION

| Conductors: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| AL — Aluminum |  | SC | - Silver Copper |  | TC | Tinned Copper |
| BC - Bare Copper |  | CCS | - Copper Covered Wire |  | SCCS - | Silver Copper, Covered Steel |

| Dielectncs: | VELOCITY %) | DIELECTRIC CONSTANT | TEMPERATURE RANGE |
| --- | --- | --- | --- |
| PTFE (Polytetrafluorethylene) | 69.0 | 2.1 | - 250°C to + 250°C |
| PE (Polyethylene) | 65.9 | 2.3 | - 65°C to + 80°C |
| FP (Foamed Polyethylene) | 78.0 | 1.4 -2.1 | - 65°C to + 80°C |
| FEP Fluorinated Ethylene Propylene) | 69.0 | 2.1 | - 70°C to + 200°C |
| Cellular FEP | 78.0 | 1.5 | - 75°C to + 200°C |
| ASP (Air Spaced Polyethylene) | 84.0 | 2.3 | - 40°C to +80°C |
| Jackets: |  |  |  |
|  |  |  | TEMPERATURE RANGE |
| PVC (Polyvinylchloride — Type 1) |  |  | -50°Cto+ 105°C* |
| NCV (Non-contaminating PVC - Type 2A) |  |  | -^OOCto+^fC * |
| PE (Polyethylene) |  |  | - 55°C to + 85° C |
| FEP (Fluorinated Ethylene Propylene) |  |  | - 70°C to + 200°C |
| FG (Fiberglass/Silicone Braid) |  |  | - 70°C to + 250°C |
| PUR (Polyurethane) |  |  | - 100°Cto+1250C * |
| PTFE (Polytetrafluoroethylene) |  |  | - 250°C to +'250°C |

SEMI-RIGID CABLE

Dielectric:   PTFE Impedance: 50 Ohm Capacitance: 29.6 pf/ft.

| OUTER DIA. | NEMAL NO. | MILITARY NO. | WEIGHT Lbs/M | MAX.OPER. VOLTAGE (RMS) | OPER.TEMP. RANGE °C | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| .141" | SR 141 | M17/130-RG402 | 34.3 | 1900 | -40to+125o | Swept 500 MHz-20 GHz |

COAXIAL CABLE PERFORMANCE SPECIFICATIONS

| RG/U | NEMAL PART NO. | NOMINAL LOSS CHARACTERISTICS db Per 100 Ft. |  |  |  |  | MAXIMUM POWER RATING Input Power (Watts) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10MHz | 50MHz | 100MHz | 400MHz | 3000MHz | 10MHz | 50MHz | 100MHz | 3000MHz |
| 6 | 1000,1001,1004,1007 | .90 | 1.32 | 1.90 | 3.80 | 17.00 | * | * | * | * |
| 6 | 1008,1010 | .90 | 1.32 | 1.90 | 3.80 | 17.00 | * | * | * | * |
| 6A | 1005 | 1.00 | 2.00 | 2.90 | 5.30 | 18.00 | * | * | * | * |
| 8 | 1100 | .66 | 1.50 | 2.20 | 4.60 | 19.00 | 3700 | 1300 | 850 | 95 |
| 8 | 1101,1102 | .55 | 1.20 | 1.80 | 3.70 | 17.00 | * | * | * | * |
| 8X | 1110 | 1.10 | 2.50 | 3.70 | 8.00 | 32.00 | 1000 | 380 | 240 | 32 |
| 9B | See RG 214 | — | — | — | — | — | — | — | — | - |
| 11 | 1200 | .66 | 1.50 | 2.20 | 4.60 | 19.00 | 2500 | 1000 | 650 | 70 |
| 11 | 1201,1202,1210 | .45 | 1.00 | 1.50 | 2.20 | N/A | * | * | * | * |
| 11A | 1205 | .66 | 1.50 | 2.20 | 4.60 | 19.00 | 2500 | 1000 | 650 | 70 |
| 14A | See RG217 |  |  |  |  |  | — | — | — | — |
| 17 | See RG218 |  |  |  | — | — | — | — | — | — |
| 22B | 1920 | 1.20 | 2.80 | 4.20 | 9.50 | N/A | 1700 | 650 | 430 | 50 |
| 55A | See RG 223 | — | — | — | — | — | — | — | - | - |
| 55B | 1401 | 1.35 | 3.00 | 4.30 | 8.80 | 36.00 | 800 | 310 | 205 | 28 |
| 58 | 1410.1735 | 1.40 | 3.00 | 4.50 | 9.50 | 38.00 | 730 | 280 | 180 | 25 |
| 58 | 1412 | 1.10 | 2.80 | 3.60 | 8.90 | 35.00 | * | * | * | * |
| 58A | 1420 | 1.40 | 3.30 | 4.90 | 11.00 | 41.00 | .650 | 255 | 170 | 22 |
| 58A | 1421 | 1.25 | 3.00 | 4.50 | 10.00 | 37.00 | * | * | * | * |
| 58C | 1425 | 1.40 | 3.30 | 4.90 | 11.00 | 41.00 | 650 | 255 | 170 | 22 |
| RG59 | 1500, 1520, 1560 | 1.10 | 2.30 | 3.30 | 6.70 | 25.50 | 1300 | 480 | 310 | 40 |
| RG59 | 1502 | 1.05 | 2.20 | 3.00 | 6.60 | 25.00 | * | * | * | * |
| RG59 | 1504,1506,1508,1530. 1540.1550,1740.1750 | .85 | 1.75 | 2.45 | 4.75 | 20.00 | * |  | * | * |
| RG59 | 1510,1512 | .80 | 1.80 | 2.60 | 5.60- | 23.00 | * | • | * | * |
| 62A | 1600,1745 | .90 | 1.90 | 2.80 | 5.20 | 18.40 | 1300 | 480 | 310 | 40 |
| 71 | 1620 | .90 | 1.90 | 2.80 | 5.20 | 18.40 | 1300 | 480 | 310 | 40 |
| 108A | 1910 | 2.30 | 5.20 | 7.50 | 16.00 | 54.00 | 340 | 145 | 100 | 15 |
| 115 | 1700 | .60 | 1.40 | 2.10 | 4.50 | 14.00 | 25000 | 9500 | 6300 | 880 |
| 122 | 1460 | 1.60 | 4.40 | 6.90 | 16.60 | 57.20 | 540 | 205 | 140 | 18 |
| 141A | 1715 | 1.20 | 2.70 | 3.90 | 8.00 | 26.00 | 9000 | 3500 . | 3400 | 350 |
| 142B | 1705 | 1.20 | 2.70 | 3.90 | 8.00 | 26.00 | 9000 | 3500 | 2400 | 350 |
| 174 | 1450 | 3.80 | 6.50 | 8.90 | 17.50 | 64.30 | 170 | 72 | 50 | 7 |
| 178A | 1710 | 5.30 | 10.00 | 13.30 | 27.50 | 78.00 | 710 | 340 | 240 | 41 |
| 179B | 1712 | 5.00 | 7.90 | 9.80 | 15.80 | 43.00 | 1600 | 780 | 570 | 110 |
| 180B | 1714 | 3.10 | 4.20 | 5.10 | 10.40 | 36.00 | 2500 | 1100 | 800 | 135 |
| 188A | 1720 | 3.80 | 7.90 | 11.50 | 20.00 | 58.00 | 1250 | 600 | 450 | 80 |
| 196A | 1725 | 5.30 | 10.00 | 13.30 | 27.50 | 78.00 | 710 | 340 | 240 | 41 |
| 213 | 1130 | .66 | 1.50 | 2.20 | 4.60 | 19.00 | 3700 | 1300 | 850 | 95 |
| 214 | 1140,1150 | .66 | 1.50 | 2.20 | 4.60 | 19.00 | 3700 | 1300 | 850 | 95 |
| 217 | 1310 | .41 | 1.00 | 1.40 | 3.10 | 13.00 | 6000 | 2000 | 1200 | 120 |
| 218 | 1800 | .23 | .56 | .81 | 1.90 | 9.00 | 18000 | 5400 | 3200 | 240 |
| 223 | 1470 | 1.35 | 3.00 | 4.30 | 8.80 | 36.00 | 800 | 310 | 705 | 28 |
| 225 | 1760 | .60 | 1.40 | 2.10 | 4.50 | 14.00 | 25000 | 9500 | 6300 | 880 |
| 316 | 1730 | 3.80 | 7.90 | 11.50 | 20.00 | 58.00 | 1250 | 600 | 450 | 80 |
| 393 | 1770 | .60 | 1.40 | 2.10 | 4.50 | 14.00 | 25000 | 9500 | 6300 | 880 |
| Twinax | 1900,1950 | 1.10 | 2.50 | 4.10 | 10.20 | N/A | - | - | - | - |
| Belden9913 | 1180 | .40 | .90 | 1.40 | 2.60 | 9.50 | 3500 | 1500 | 1000 | 110 |
| Belden8281 | 1185 | .78 | 2.20 | 2.70 | 5.50 | N/A | - | - | - | - |
| Belden8218 | 1190 | 1.70 | 4.20 | 5.70 | 12.10 | N/A | - | - | - | - |
| Triax RG 11 | 1820 | .70 | 1.00 | 1.50 | 3.30 | N/A | 3200 | 1700 | 750 | N/A |
| Triax RG58 | 1830 | 1.70 | 3.30 | 4.90 | 12.00 | N/A | 800 | 420 | 220 | N/A |
| Triax RG59 | 1840 | 1.20 | 1.80 | 2.60 | 5.60 | N/A | 1300 | 700 | 360 | N/A |

* Foam Dielectric Cables have approximately 35-50% less power handling capability than similar polyethylene dielectric cables. All ratings are subject to variation with VSWR, ambient temperatures, and altitude.